' Clock Defines 
' ---------------- 
DS_SCL VAR PORTC.3 	' I2C clock pin 
DS_SDA VAR PORTC.4 	' I2C data pin 
RTC CON %11010000 	' RTC device address (byte addressing) 

'Setting the RTC so we can get out 1Hz tick output 
'------------------------------------------------- 
' -------------- RTC definitions ----------------- 
SecReg 	CON $00 	' seconds address (00 - 59)

‘ MSB of SecReg must be set to a 0 to enable RTC

MinReg CON $01 	' minutes address (00 - 59) 
HourReg CON $02 	' hours address (01 - 12) or (00 - 23) 
DayReg CON $03 	' day address (1 - 7) 
DateReg CON $04 	' date address (01 - 28/29, 30, 31) 
MonthReg CON $05 	' month address (01 - 12) 
YearReg CON $06 	' year address (00 - 99) 
ContReg CON $07 	' control register 
RTCflag CON 0 		' RTC flag (location 0 of internal EEPROM) 
RTCset VAR BIT 	' bit to check if RTC has been set 
cntrl CON %00010000 	' sets the SQW/OUT to 1Hz pulse, logic level low 

' The variable below holds the values entered: 
' entered by the user 

_sec 	VAR BYTE ' $00 set seconds 
_min 	VAR BYTE ' $56 set minutes 
_hr 	VAR BYTE ' $23 set hour 
_day 	VAR BYTE ' $06 set day 
_date 	VAR BYTE ' $28 set date 
_mon 	VAR BYTE ' $02 set month 
_yr 	VAR BYTE ' $03 set year 



sec 	VAR BYTE ' seconds 
MINs 	VAR BYTE ' minutes 
hr 	VAR BYTE ' hours 
day 	VAR BYTE ' day 
date 	VAR BYTE ' date 
mon 	VAR BYTE ' month 
yr 	VAR BYTE ' year 
' 
' Initialise Hardware 
' ------------------- 
‘******Put your particular TRIS settings here*******

Pause 1000 
LCDOut I,Clr:Pause 30 
LCDOut I,Line1+2," RTC TEST " 
LCDOut I,Line2+2,"..Power On.. !!" 

GoSub SetTimeAndDate 

' Main Program Loop 
' ----------------- 
Loop: 


I2CRead DS_SDA, DS_SCL, RTC, SecReg, [sec,MINs,hr,day,date,mon,yr] 
' Read Ds1307 
' Display Section 
' --------------- 
LCDOut I,Clr:Pause 30 	' Clear Lcd 
LCDOut I,Line1,"Time: ",HEX2 hr, ":", HEX2 MINs, ":", HEX2 sec 
LCDOut I,Line2,"Date : ",HEX2 date,"-",HEX2 mon,"-",HEX2 yr 


Pause 500 ' Allow Time for user to view 
GoTo Loop 

End 



SetTimeAndDate: 
'============= 
_yr=0:_date =0:_mon =0:_day=0:_hr=0:_min=0:_sec=0 

' The constants below set the RTC to: 16:30:00 on 13-07-2003 
_yr=$03 
_mon=$07 
_date=$13 
_hr=$16 
_sec=$00 
_min=$30 

I2CWrite DS_SDA, DS_SCL, RTC, SecReg,[_sec,_min,_hr,_day,_date,_mon,_yr,cntrl] 
Pause 10 
Return