' Program Inkeys12.Inc

' *************************************************************

' * For use with EXPERIMENTING WITH THE PICBASIC PRO COMPILER *

' *							                                  *

' *  This source code may be freely used within your own      *

' *  programs. However, if it is used for profitable reasons, *

' *        please give credit where credit is due.	          *

' *  And make a reference to myself or Rosetta Technologies   *

' *			Ryan S. Leis			                	      *

' *			Les. Johnson			                          *

' *************************************************************

'

' This is an include file that integrates the INKEYS12 subroutine at the start

' of the host program.

' ** Declare Subroutine Variables **

Key		    Var	byte	BANK0	SYSTEM	      ' Key pressed value

Key1		    Var	byte	BANK0	SYSTEM

keydata		Var	Byte	BANK0	SYSTEM

numlok		Var	bit	BANK0	SYSTEM

'flagm       Var	byte    BANK0	SYSTEM

'keypress	Var	byte	BANK0	SYSTEM

Debounce 	Var 	Byte	BANK0	SYSTEM 	' Flag to indicate a keypress.

flagA       VAR     bit    BANK0	SYSTEM                ' Flag to hold the number of keystrokes

flagB       VAR     Byte    BANK0	SYSTEM

flagC       VAR     bit    BANK0	SYSTEM

'	flagD       VAR     Byte    BANK0	SYSTEM

flagk       VAR     Byte    BANK0	SYSTEM

Inc_Flag	Var	    Bit			            ' Internal Flag for detecting a key press

Inc_D_Flag 	Var 	Bit 			        ' Debounce flag used by Inkeys.

'timer0_flag Var     Word	BANK0	SYSTEM

'kpress_flag Var     Word	BANK0	SYSTEM

'%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

'flagl = 0

' flagm = 0

keydata = 0

'keypress = 0

Inc_D_Flag=0

numlok = 0

' Reset the debounce flag, prior to calling the subroutine

Goto Over_Inkeys12					' Jump Over the Subroutine

Gosub Inkeys			' Do the keyboard scanning

Inkeys:

Debounce=1

'flagl = 0				' Setup the initial value for Debounce

'flagm = 0

flagk = 0

Key = 0					' Clear the variable KEY, prior to scanning

Key1 = 0

'keypress = 0

trisA = %000001

'TrisA.5=0:TrisA.1=0			' Make the first four bits of PortA Outputs

'TrisA.2=0:TrisA.3=0

'TrisA.0=0 '1

trisB = %11100000

'TrisB.5=1:TrisB.6=1			' Make the last Three bits of PortB inputs

'TrisB.7=1

Option_Reg.7=0				' Enable Internal PortB Pullup Resistors

'flagl = flagl+1

PortA=%011111				' Pull the first Column line LOW

'PortA=%1110

Gosub ScanRow				' Scan the Rows

If Inc_Flag=1 then goto Map		' If a key is pressed then map it

PortA=%111101				' Pull the second Column line LOW

Gosub ScanRow				' Scan the Rows

If Inc_Flag=1 then goto Map		' If a key is pressed then map it

PortA=%111011				' Pull the third Column line LOW

Gosub ScanRow				' Scan the Rows

If Inc_Flag=1 then goto Map		' If a key is pressed then map it

PortA=%110111				' Pull the fourth Column line LOW

Gosub ScanRow				' Scan the Rows

If Inc_Flag=1 then goto Map		' If a key is pressed then map it

Inc_D_Flag=0				' No key pressed, so Reset debounce flag

Debounce=0 			        ' No key pressed, so Reset key Debounce flag

Goto Exit				' Exit from the subroutine

' Do the following code if a key has been pressed

Map:

if numlok = 1  then

flagb = 0

goto Map1 ' numbers only!!!

endif

if Inc_D_Flag <> flagA then  'flag to count the number of Inc_D_Flag equals to 1 upon transition from 0 to 1

if Inc_D_Flag = 1 then flagB = flagB+1   'edge detection algorithm

if flagB > 9 then flagB = 1

flagA = Inc_D_Flag

endif

'****************keypress row routine************* tells a newly pressed row*****

If Inc_D_flag = 1 then

if keydata <> flagk then

flagb = 1

keydata = flagk

endif

goto   Exit

endif

'********************************************************************************

Map1:

If Inc_D_Flag = 1 then  Exit ' Already responded to this press, so exit

Inc_D_Flag = 1  '1				' Set Debounce flag

Debounce=0 				' Reset key Debounce flag

Exit:	'Lookup Key,["1","2","3","4","5","6","7","8","9"],Key ' Map of the keypad legends for numeric output

' **To convert the output to an ascii value comment the line above and uncomment the line below

If flagb = 0 then Lookup Key,[1,2,3,4,5,6,7,8,9,0,0],Key1

If flagb = 1 then Lookup Key,["1","2","3","4","5","6","7","8","9"," ","0"," "],Key1 ' Map of the keypad legends for ascii output

If flagb = 2 then Lookup Key,[".","A","D","G","J","M","P","T","W"," ","*"," "],Key1 ' Map of the keypad legends for numeric output

If flagb = 3 then Lookup Key,[",","B","E","H","K","N","Q","U","X"," ","#"," "],Key1 ' Map of the keypad legends for numeric output

If flagb = 4 then Lookup Key,["?","C","F","I","L","O","R","V","Y"," ","^"," "],Key1 ' Map of the keypad legends for numeric output

If flagb = 5 then Lookup Key,["!","a","d","g","j","m","S","t","Z"," ","_"," "],Key1 ' Map of the keypad legends for numeric outpu

If flagb = 6 then Lookup Key,["'","b","e","h","k","n","p","u","w"," ","{"," "],Key1

If flagb = 7 then Lookup Key,["@","c","f","i","l","o","q","v","x"," ","}"," "],Key1

If flagb = 8 then Lookup Key,[":","+", 34,"<","(","$","r","%","y"," "," "," "],Key1

If flagb = 9 then Lookup Key,[";","-","/",">",")","&","s","|","z"," "," "," "],Key1

'lookup flagl,["a","b","c","d","e","f","g","h","i","j","k","l","m"],flagm

Return					  ' Exit from the subroutine

' This subroutine scans the rows

' The bit, "FLAG" returns a 1 if a key is pressed, and 0 if no key pressed

' Also, if no key is pressed the variable "KEY" will return with the value of 12

ScanRow:

Inc_Flag=1				  ' Set Flag initially to 1

If PortB.5=0 then S_Exit  ' Return if a key on the first row is pressed

Key=Key+1				  ' Else increment KEY, and try another row

flagk = flagk+1

'flagl = flagl+1

If PortB.6=0 then S_Exit  ' Return if a key on the second row is pressed

Key=Key+1				  ' Else increment KEY, and try another row

flagk = flagk+1

'flagl = flagl+1

If PortB.7=0 then S_Exit  ' Return if a key on the third row is pressed

Key=Key+1				  ' Else increment KEY, KEY now equals 12

flagk = flagk+1

'flagl = flagl+1

Inc_Flag=0				  ' Set the flag to indicate no key pressed

S_Exit:	Return				  ' And exit the subroutine

Over_Inkeys12: