'ryansleis leisryan@yahoo.com

; @ include P16F877A.INC

; @ _config _HS_OSC & _WDT_OFF & _PWRTE_ON & _BODEN_OFF & _LVP_OFF & _CP_OFF & _CPD_OFF

;        @ DEVICE PIC16F877A, HS_OSC

;        @ DEVICE PIC16F877A, WDT_ON

;        @ DEVICE PIC16F877A, PWRT_ON

;        @ DEVICE PIC16F877A, BOD_ON

;        @ DEVICE PIC16F877A, LVP_OFF

;        @ DEVICE PIC16F877A, CPD_OFF

;        @ DEVICE PIC16F877A, PROTECT_OFF

;        @ DEVICE PIC16F877A, WRT_OFF

DEFINE OSC		12		' Set the Xtal Frequency to 12 mHz

Include "Modedefs.bas"

Include "inkeys_sms.inc"			' Load in the keypad scan subroutine

'    Include "LCDbar_INC.bas"

INCLUDE "DT_INTS-14.bas"     ' Required

INCLUDE "ReEnterPBP.bas"     ' Include if using PBP interrupts

'    Include "Sony_RX.inc"

'	Hardware Defines

'	----------------

'

'   RS2332 setup!!!

TRISA = %00000001

TRISB = %11100000

TRISC = %10000000

TRISD = %00000000

' 	LCD Display

'	-----------

DEFINE LCD4X20

DEFINE LCD_DREG	       	PortD		' Set LCD Data to PortD

DEFINE LCD_DBIT	       	0		    ' Set starting Data to Bit0

DEFINE LCD_EREG	       	PortD		' Set LCD Enable to PortC

DEFINE LCD_EBIT        	5		    ' Set LCD Enable line to PortC.1

DEFINE LCD_RSREG       	PortD		' Set LCD Register Select to PortB

DEFINE LCD_RSBIT       	4		    ' Set LCD RS line to PORTC.0

DEFINE LCD_BITS	       	4		    ' Set for a 8 bit Bus

DEFINE LCD_LINES       	4		    ' Set number of lines to 4

DEFINE  LCD_COMMANDUS	2000	' Command Delay time in uS was 2000

DEFINE  LCD_DATAUS		50		' Data Delay time in uS    was 50

'	Serial_Communications

'	--------------------------

'DEFINE  HSER_BAUD 19200

'DEFINE  HSER_CLROERR 1  ' Automatically clear over-run errors

'DEFINE  HSER_RCSTA 90h	' Enable USART receive

'DEFINE  HSER_TXSTA 24h	' TXSTA=%00100100. TX enable, BRGH=1 for high-speed

RCSTA = $90 ' Enable serial port & continuous receive

TXSTA = $24 ' Enable transmit, BRGH = 1

SPBRG = 38 ' 19200 Baud @ 0.16%

OERR    VAR RCSTA.1     ' Alias USART over-run bit

CREN    VAR RCSTA.4     ' Alias USART continuous receive enable bit

RCIF    VAR PIR1.5      ' Alias USART received character interrupt flag bit

'	AD_Configurations

'	--------------------------

Define	ADC_BITS	10		' Set resolution of conversion

Define	ADC_CLOCK	2		' Set clock source (x/FOSC)

Define	ADC_SAMPLEUS	10		' Set sampling time (in uS)                  ' Right Justify AD result

ADFM		Var	ADCON1.7	' ADC result format select bit

PCFG3		Var	ADCON1.3	' ADC port configuration bit

PCFG2		Var	ADCON1.2	' ADC port configuration bit

PCFG1		Var	ADCON1.1	' ADC port configuration bit

PCFG0		Var	ADCON1.0	' ADC port configuration bit

PCFG0=0

PCFG1=1

PCFG2=1

PCFG3=1					' Configure for AN0 as analogue input

ADFM=1					' Right justified result in ADRESL and ADRESH

'	Buttons

'	---------------------------

I		    Con	254		' Control Byte

Clr		    Con	1		' Clear the display

Line1		Con	128		' Point to beginning of line 1

Line2		Con	192		' Point to beginning of line 2

Line3		Con	148		' Point to beginning of line 3

Line4		Con	212		' Point to beginning of line 4

Cgram		Con	64		' Point to Cgram within LCD

Shift_L		Con	24		' Shift display left

Shift_R		Con	28		' Shift display right

Cursor       Con 13      ' cursor

Cursor_L     Con 16

Cursor_R     Con 20

'_____________________________________________________________________________

'ports setup

'trisB = %11100000

'_____________________________________________________________________________

'	Buttons

'	-------

'

'	Software Defines

'	----------------

'PS1    VAR     T1CON.5

'PS0    VAR     T1CON.4

AMPMFlag var byte		' Flag for 12/24 Hour Mode 0=24, 1=12

' - Yes I know it's a BYTE but I can't be bothered

ButtonPress var BYTE		' Button Counter Variable

DataA var BYTE

Hours var BYTE

Days var BYTE

Hundredths var BYTE

MenuStep var BYTE		' Status Indicator for Menu Display

Minutes var BYTE

Seconds var BYTE

TMR1Cal	var BYTE		' Timer Calibration Value

TMR1CalAR var BYTE		' Timer Advance/Retard Indicator

TMR1RunOn var Word		' variable holding TMR1 Run-On value

lcd_char        Var   Word

keypress	    Var	  Word

smoothed        Var   Word

Value2          Var   Word

GP              Var  byte

Dy             VAR   byte

Dy1            VAR   byte

Dy2            VAR   byte

Hr             VAR   byte

Mi             VAR   byte

Sec            VAR   byte

Value            VAR   byte

Value1           VAR   byte

total            VAR   byte

skednum          var   byte

skedvar          var   byte

eenum            var   byte

ind              Var   byte

dy_diff          Var   byte

hr_diff          Var   byte

mi_diff          Var   byte

skedvar_i        Var   byte

liner         Var   byte

Out              Var   byte

timeset          Var   bit

heat_flag        Var   byte

ByteStore var byte[4]

BytesIn var byte[4]       '12 bytes in bytes array

BytesReply var byte [9]

BytesPhone var byte[20]

portb = 0

clear

MobilePhoneSet:

for ind  = 0 to 35          '********AT command sending limit each character to 1ms

Lookup ind, ["A","T","E","0",13,10,"A","T","+","C","M","G","F","=","1"_

,13,10,"A","T","+","C","N","M","I","=","1",44,"2",44,"0",44,"0",44,"0"_

,13,10],lcd_char

',"A","T","+","I","P","R","=","9","6","0","0",13,10]     'baudrate at command

HSerout [lcd_char]

pause 50

next ind

CREN = 0                        ' Disable continuous receive to clear overrun flag

CREN = 1                        ' Enable continuous receive

'

'	EEPROM Settings

'	---------------

Data @0,1			' Advance/Retard Indicator 'RETARD

Data 75				' Calibration Value

Data 0				' 12/24 Hour Flag

'

'	Software Constants

'	------------------

TMR1CalMax con 100		' Maximum adjustment (+/-100uS per 10mS interrupt)

TMR1Preset con  $8B32 '$D910		' 10mS Timer Reload value, offset by 20uS

'

'	Initialise Main Program

'	-----------------------

'    	gosub inkeys

'    	Pause 1000			' Timeout for LCD Hardware to wake-up...

Read 0,TMR1CalAR		' Read Calibration Advance/Retard Indicator

Read 1,TMR1Cal			' Read Calibration Value

Read 2,AMPMFlag			' It's what it says...

Hundredths = 0			' Reset Timer Counter variables

Seconds = 0

Minutes = 0

Hours = 0

'

'	Initialise TMR1 & USART Interrupts

'	--------------------------

'    	Gosub SetTimer			' Set the Timer for next 10mS Interrupt

'    	PIE1.0 = 1			' Enable TMR1 Interrupts

'    	PIE1.5 = 1          ' Enable USART receive Interrupts RCIE

'    	INTCON.6 = 1			' Enable all unmasked Interrupts

'    	INTCON.7 = 1			' Enable Global Interrupts

ASM

INT_LIST       macro    ; IntSource,        Label,  Type, ResetFlag?

INT_Handler    RX_INT,      _USART_ISR,    PBP,  yes

INT_Handler   TMR1_INT,     _TickCount,   PBP,  yes

endm

INT_CREATE               ; Creates the interrupt processor

;    INT_ENABLE   RX_INT     ; enable USART  interrupts

;    INT_ENABLE   TMR1_INT

ENDASM

Gosub SetTimer			' Set the Timer for next 10mS Interrupt

'    	 On Interrupt goto TickCount

DisplayRestart:

LCDOut $FE,1			' Clear Display

LCDOut $FE,$0C

'

'	Main Program Loop

'	-----------------

@    INT_ENABLE   RX_INT

DisplayLoop:

'        Lcdout I,201,"Menu",I,Line3,"1=am/p 2=Sked 3=Loop",I,Line4,"4=Revue 5=Temp #=Clk"

'        Lcdout I,Line3,"1=am/p 2=Sked 3=Loop",I,Line4,"4=Rvue 5=Temp 6=SMS"

If AMPMFlag=0 then

PortD.6 = 0

'

'	Here we have got a 24 Hour Clock Display

'	----------------------------------------

LCDOut I,$85,DEC2 Days,"-",DEC2 Hours,":",DEC2 Minutes,":",DEC2 Seconds

else

PortD.6 = 1

'

'	and here we have a 12 Hour Clock Display

'	----------------------------------------

LCDOut I,$85

DataA=Hours

If DataA=0 then DataA=12

If DataA>12 then DataA=DataA-12

If DataA<10 then LCDOut " "

LCDOut DEC2 Days,"-",#DataA,":",DEC2 Minutes,":",DEC2 Seconds," "

'Lcdout I,Line3,"Press keys for menu"

If Hours>11 then

LCDOut "PM"

else

LCDOut "AM"

endif

endif

Lcdout I,200,BIN5 PortB, I,Line3,"1=am/p 2=Sked 3=Loop",I,Line4,"4=Rvue 5=Temp 6=SMS"

'

'	Check For Button Press

'	----------------------

''@    INT_DISABLE   TMR1_INT

Gosub schedule

Gosub skedrevue

Gosub GetButton

Gosub alarm

Gosub loop

Gosub Dim_Graph

Gosub Temperature

Gosub SMS_Setup

Gosub Out_Setup

@    INT_ENABLE   TMR1_INT

''@    INT_ENABLE   RX_INT

'

'	Toggle 24 Hour Mode

'	-------------------

If ButtonPress=1 then

AMPMFlag=AMPMFlag^1

Write 2,AMPMFlag

Goto DisplayRestart

endif

If ButtonPress<>2 then goto DisplayLoop

'    	Disable

'

'	Setup Mode

'	----------

'LCDOut $FE,"Set Time Please:"

LCDOut I,1,I,130,"Set Time Please:"

Lcdout I,$0E                        '********************cursor underline

Lcdout I,215,"Press # to set"

LCDOut I,197,DEC2 Days,"-",DEC2 Hours,":",DEC2 Minutes,":",DEC2 Seconds ,I,197

MenuStep=0

'@    INT_ENABLE   TMR1_INT

MenuLoop:

Gosub GetButton

'

'	SET Mode

'	--------

If ButtonPress=2 then

'timeset = 1

MenuStep=MenuStep+1

If MenuStep=4 then

timeset = 0

LCDOut I,1,I,130,"JRU Mandaluyong",$FE,195,"Multifunction",I,148,"SMS Control & Timer",$FE,214,"Have a Nice Day"

'LCDOut I,$FE,$83,"Thank You",$FE,$C0,"Have a Nice Day"

'Pause 2000

'Pause 200

Goto DisplayRestart

endif

endif

'If ButtonPress=1 then

timeset = 1

'	Hours

'	-----

If MenuStep=0 then

Days = dy

If Days>30 then

Days=0

Value = 0

Value1 = 0

endif

LCDOut I,197,DEC2 Days,I,197

endif

If MenuStep=1 then

Hours = Hr

If Hours>23 then

Hours=0

Value = 0

Value1 = 0

endif

LCDOut I,200,DEC2 Hours,I,200'$FE,$C4

endif

'

'	Minutes

'	-------

If MenuStep=2 then

Minutes = Mi

If Minutes>59 then Minutes=0

LCDOut I,203,DEC2 Minutes,I,203'$FE,$C7

endif

'

'	Seconds

'	-------

If MenuStep=3 then

Seconds = Sec

If Seconds>59 then Seconds=0

LCDOut I,206,DEC2 Seconds,I,206'$FE,$CA

endif

Goto MenuLoop

'************reserve this code space for clock setup!!!!!!!!!!!***********************************

Out_Setup:

gosub inkeys

if Inc_D_Flag = 1 then

'            numlok = 1

if key = 10 then

PortB.0 = PortB.0^1

pause 200

endif

if key = 9 then

PortB.1 = PortB.1^1

pause 200

endif

if key = 8 then

PortB.2 = PortB.2^1

pause 200

endif

if key = 7 then

PortB.3 = PortB.3^1

pause 200

endif

if key = 6 then

PortB.4 = PortB.4^1

pause 200

endif

endif

Return

GetButton:

gosub inkeys

ButtonPress=0

if Inc_D_Flag <> flagC then  'flag to count the number of Inc_D_Flag equals to 1 upon transition from 0 to 1

if Inc_D_Flag = 0 then value1 = value   'edge detection algorithm

if Inc_D_Flag = 1 then value = key1

flagC = Inc_D_Flag

endif

if Inc_D_Flag = 1 then

numlok = 1

if key <> 11 then

Value = key1

total = (Value1*10) + value

if total > 59 then

total = 0

Value = 0

Value1 = 0

endif

'keypress = 0

'Value = key1

endif

if key = 11 then

total = 0        ' set the total value every menustep to "0"

buttonpress = 2

'keypress = keypress + 1

pause 200

endif

if key = 9 then

menustep = 0

total = 0

pause 100

endif

If key = 0 then

buttonpress = 1

pause 200

endif

endif

if timeset = 1 then

if menustep = 0 then

if total > 30 then

total = 0

Value = 0

Value1 = 0

endif

dy = total

endif

if menustep = 1 then

if total > 23 then

total = 0

Value = 0

Value1 = 0

endif

Hr = total

endif

if menustep = 2 then Mi = total

if menustep = 3 then Sec = total

if Menustep =>5 then menustep = 0

endif

Return

'************reserve this code space for schedule setup!!!!!!!!!!! ********************************

Schedule:

gosub inkeys

if key =  1 then

if timeset = 1 then return

'if pinflag = 1 then gosub pinlock

'pause 200

out = 0

total = 0

Value = 0

Value1 = 0

numlok = 1

keypress = 0

skednum = 0

dy = 0

Hr = 0

Mi = 0

Lcdout I,Clr,I,line1,"Sked# Day-Time: Out#",I,213,"new = *  save = #",I,$0E,I,$C0,"#00 <00-00:00> 00000",I,193   'I,Line1,"med scheduler setup",

'Lcdout I,$C4,DEC2 Hours,":",DEC2 Minutes,":",DEC2 Seconds,$FE,$C4

'Lcdout I,$C0,"sked #",DEC2 skednum," time ",DEC2 Hours,":",DEC2 Minutes,I,C7',":",DEC2 Seconds

keypress = 0

pause 300

goto schedset

else

return

endif

'

schedset:

gosub inkeys

if Inc_D_Flag <> flagC then  'flag to count the number of Inc_D_Flag equals to 1 upon transition from 0 to 1

if Inc_D_Flag = 0 then value1 = value   'edge detection algorithm

if Inc_D_Flag = 1 then value = key1

flagC = Inc_D_Flag

endif

if Inc_D_Flag = 1 then

if key = 11 then

keypress = keypress + 1

total = 0        ' set the total value every menustep to "0"

pause 200            '****to be clock timed later comparing the difference in lapsed time

endif

if key <> 11 then

'if keypress <> 4 then

Value = key1

total = (Value1*10) + value

if total > 59 then

total = 0

Value = 0

Value1 = 0

endif

'Value = key1

'endif

endif

Lcdout I,193,DEC2 skednum,I,197,DEC2 dy,I,200,DEC2 hr,I,203,DEC2 mi,I,207_

,bin1 out.4,I,208,bin1 out.3,I,209,bin1 out.2,I,210,bin1 out.1,I,211_

,bin1 out.0,I,193     '" time ",DEC2 Hr,":",DEC2 Mi,I,$C7',":",DEC2 Sec

skedvar = (skednum*4)   '**********ensuring eeprom correct write space!!!

if keypress = 0 then

if total > 24 then total = 0

skednum = total

LCDOut $FE,193

endif

if keypress = 1 then

if total > 30 then total = 0

dy = total

LCDOut $FE,197

endif

if keypress = 2 then

if total > 23 then total = 0

Hr = total

LCDOut $FE,200

endif

if keypress = 3 then

Mi = total

LCDOut $FE,203

endif

if keypress = 4 then

total =  value

'if total > 1 then total = 0

out.4 = total

LCDOut $FE,207

endif

if keypress = 5 then

total =  value

'if total > 1 then total = 0

out.3 = total

LCDOut $FE,208

endif

if keypress = 6 then

total =  value

'if total > 1 then total = 0

out.2 = total

LCDOut $FE,209

endif

if keypress = 7 then

total =  value

'if total > 1 then total = 0

out.1 = total

LCDOut $FE,210

endif

if keypress = 8 then

total =  value

'if total > 1 then total = 0

out.0 = total

LCDOut $FE,211

' out = (out1*16) + (out2*8) + (out3*4) + (out4*2) + out5

endif

if keypress = 9 then

if skednum <> 0 then

write skedvar, skednum

write (skedvar+1), dy

write (skedvar+2), Hr

write (skedvar+3), Mi

write (skednum+99), out

Lcdout I,149,"sched #",#skednum," saved!!",I,211

'pause 1000

'Lcdout I,Clr,I,line1,"Sked# Day-Time: Out#",I,213,"new = *  save = #"

endif

Lcdout I,211

endif

if key = 9 then

if keypress = 1 then              '***************if keypress = 4 then

if skednum <> 0 then

write skedvar, $FF

write (skedvar+1), $FF

write (skedvar+2), $FF

write (skedvar+3), $FF

write (skednum+99), $FF

Lcdout I,149,"sched #",#skednum,"deleted",I,193

'pause 1000

endif

endif

skednum = 0

keypress = 0

total = 0

dy = 0

hr = 0

mi = 0

Lcdout I,193

pause 200

endif

'Lcdout I,$C0,"sked #",DEC2 skednum," time ",DEC2 Hr,":",DEC2 Mi,I,$C7

endif

if keypress = 10 then

Lcdout I,Clr,I,$0C

pause 500

return

endif

goto schedset

'******************** reserve this code space for reviewing schedule ********************************

skedrevue:

gosub inkeys

if key =  3 then

if timeset = 1 then return

'if pinflag = 1 then gosub pinlock

pause 300

out = 0

total = 0

ind = 0

skedvar = 0

numlok = 1

keypress = 0

Lcdout I,Clr,I,$0E,I,130,"Schedule Viewer",I,214,"down = *  up = 0"',I,$C0,"sked # time :",I,153,"#"

read $7E ,Dy1 : read $7F ,Dy2

goto skedrev

else

return

endif

'

skedrev:

gosub inkeys

if Inc_D_Flag = 1 then

if key = 0 then keypress = 1

if key = 9 then

keypress = Keypress - 1

if keypress = 0 then keypress = 24

pause 200

endif

if key = 10 then

keypress = Keypress + 1

pause 200

endif

if keypress > 24 then keypress = 1

skedvar = 4*keypress

'for ind = 1 to 5

'if ind = 1 then

eenum = skedvar  '*****replace 4 with var dependent on keypress

read eenum ,total

if total > 24 then total = 0

skednum = total

'endif

'if ind = 2 then

eenum = skedvar+1

read eenum ,total

if total > 30 then total = 0

dy = total

'endif

'if ind = 3 then

eenum = skedvar+2

read eenum ,total

if total > 23 then total = 0

hr = total

'endif

'if ind = 4 then

eenum = skedvar+3

read eenum ,total

if total > 59 then total = 0

mi = total

'endif

'if ind = 5 then

eenum = keypress + 99

read eenum ,total

if total > 31 then total = 0

out = total

'endif

lcdout I,130,"Schedule Viewer",I,214,"down = *  up = 0",I,192,"loop#"_

,DEC2 dy1,"-",DEC2 dy2," d",DEC2 dy,"t",DEC2 Hr,":",DEC2 Mi,I,148,"sked#"_

,DEC2 keypress,I,156,"Out#<",bin5 out,">",I,154

'next ind

endif

'    gosub inkeys

if key = 11 then

Lcdout I,Clr,I,$0C

pause 500

return

endif

goto skedrev

'**************************** reserve code space for alarm!!!***********************

alarm:

out = 0

sec = seconds

'Days = dy1

read $7E ,Dy1 : read $7F ,Dy2

total = Dy2 - Days

if total = 0 then Days = Dy1

for skedvar_i = 1 to 24

read (skedvar_i+99), out

skedvar = 4*skedvar_i

for ind = 1 to 4     '2 to 3

if ind = 1 then

eenum = skedvar  '***** replace 4 with var dependent on keypress

read eenum ,skednum

endif

if ind = 2 then

eenum = skedvar+1

read eenum ,total

if total > 30 then total = 0

dy = total

endif

if ind = 3 then

eenum = skedvar+2

read eenum ,total

if total > 23 then total = 0

hr = total

endif

if ind = 4 then

eenum = skedvar+3

read eenum ,total

if total > 59 then total = 0

mi = total

endif

next ind

dy_diff = days - dy

hr_diff = Hours - Hr

mi_diff = Mi - Minutes

if skednum <> $FF then

if dy_diff = 0 then

if hr_diff = 0 then 'and mi_diff = 0 and sec = 0 then 'then

if mi_diff = 0 then

if sec = 0 then

'lcdout I,clr,I,Line1,"Multi-Function Timer",I,196,"In-Operation"',I,151,"Stand-by Pls!"

portb = out

pause 500

seconds = seconds + 2

Lcdout I,Clr,I,$0C

return

endif

endif

endif

endif

endif

next skedvar_i

return

'**************************** reserve code space for Temperature Bar-Graph***********************

Dim_Graph:

gosub inkeys

if key = 4 then

if timeset = 1 then return

'if pinflag = 1 then gosub pinlock

pause 300

numlok = 1

keypress = 0

read 124,keypress

Lcdout I,Clr,I,130,"THERMOSTAT SETUP",I,214,"down = *  up = 0"

goto Dimmer

'else

'return

endif

return

Dimmer:

if Inc_D_Flag = 1 then

if key = 0 then keypress = 0

if key = 1 then keypress = 60

if key = 2 then keypress = 120

if key = 9 then

keypress = Keypress - 1

if keypress > 120 then keypress = 0

pause 150

endif

if key = 10 then

keypress = Keypress + 1

if keypress > 120 then keypress = 120

pause 150

endif

Lcdout I,201, DEC3 keypress

;syntax-BARgraph  Value, Row, Col, Width, Range, Style

'           @     BARgraph _keypress,  3,   2,  18,  120,  lines

endif

gosub inkeys

if key = 11 then

Lcdout I,Clr,I,$0C

write 124,Keypress

pause 500

return

endif

goto dimmer

'************************************** Temperature *************************************************

Temperature:

ADCIN  0, Value2

Value2 = Value2-31

for ind = 0 to 4

if PortB.0(ind) = 1 then

Value2 = Value2 - 1

endif

next ind

if Value2 > smoothed then

smoothed = smoothed + (Value2 - smoothed)/5

else

smoothed = smoothed - (smoothed - Value2)/5

endif

'syntax- BARgraph   Value, Row, Col, Width, Range, Style

'@         BARgraph  _Value,   3,   0,    18,   255,  lines

'                 @     BARgraph _Value2,  2,   3,  17,  120,  Boxed

'               smoothed = smoothed - 30

Lcdout I,line1,#smoothed,"C  "

'                Lcdout I,line1,#Value2,"C  "

read 124,total

if smoothed => total then

eenum = 131

Value = 0

for ind  = 1 to 9

read eenum ,lcd_char

eenum = eenum + 1

BytesReply[Value] = lcd_char

Value = Value + 1

next ind

'heat_flagbit = 1

goto Heat_Transmit

else

heat_flag = 0

'heat_flagbit = 0

endif

return

'***************************Timer Loop setup*******************************************

Loop:

gosub inkeys

if key = 2 then

if timeset = 1 then return

'if pinflag = 1 then gosub pinlock

pause 300

numlok = 1

keypress = 0

total = 0

Value = 0

Value1 = 0

read $7E ,Dy1 : read $7F ,Dy2

Lcdout I,Clr,I,$0E,I,132,"Loop Control",I,212,"change = * save = #",I,193,"Day-",DEC2 Dy1,I,200,"upto Day-",DEC2 Dy2,I,197

'Lcdout I,193,"Day-",DEC2 Dy,I,200," upto Day-",DEC2 Dy2,I,197

goto Looper

'else

'return

endif

return

Looper:

gosub inkeys

if Inc_D_Flag <> flagC then  'flag to count the number of Inc_D_Flag equals to 1 upon transition from 0 to 1

if Inc_D_Flag = 0 then value1 = value   'edge detection algorithm

if Inc_D_Flag = 1 then value = key1

flagC = Inc_D_Flag

endif

if Inc_D_Flag = 1 then

if key = 11 then

keypress = keypress + 1

total = 0        ' set the total value every menustep to "0"

pause 200            '****to be clock timed later comparing the difference in lapsed time

endif

if key <> 11 then

'if keypress <> 4 then

Value = key1

total = (Value1*10) + value

if total > 30 then

total = 0

Value = 0

Value1 = 0

endif

'Value = key1

'endif

endif

Lcdout I,193,"Day-",DEC2 Dy1,I,200,"upto Day-",DEC2 Dy2',I,197',DEC2 hr,I,203,DEC2 mi,I,207,bin1 out1,I,208,bin1 out2,I,209,bin1 out3,I,210,bin1 out4,I,211,bin1 out5,I,193     '" time ",DEC2 Hr,":",DEC2 Mi,I,$C7',":",DEC2 Sec

if keypress = 0 then

'if total > 31 then total = 0

Dy1 = total

LCDOut I,197

'write $7E ,Dy1

endif

if keypress = 1 then

'if total > 31 then total = 0

Dy2 = total

LCDOut I,209

'if dy2<dy1 then dy2 = 0

'write $7F ,Dy2

endif

if keypress = 2 then

'if total > 31 then total = 0

write $7E ,Dy1 : write $7F ,Dy2

'Dy2 = total

LCDOut I,152,"Loop Saved!!"

endif

if key = 9 then

keypress = 0

Dy1 = 0

Dy2 = 0

LCDOut I,152,"Loop deleted",I,197

endif

endif

'    gosub inkeys

if keypress = 3 then

Lcdout I,Clr,I,$0C

pause 500

return

endif

goto looper

'***************************SMS_Setup setup*******************************************

SMS_Setup:

gosub inkeys

if key = 5 then

if timeset = 1 then return

Lcdout I,Clr,I,Cursor,I,129,"SMS & Outlet Setup",_       'cel no. entry space!!!

I,194,"Cel#<",I,211,">",_

I,148,"1.",I,155,"2.",I,162,"3.",_                'Outlet name entry space!!!

I,212,"4.",I,219,"5.",I,226,"6.",I,199

eenum = 128

liner = 199

for ind  = 1 to 12

read eenum ,lcd_char

lcdout I,liner,lcd_char

liner = liner + 1

eenum = eenum + 1

'pause 10

next ind

eenum = 140

liner = 150

for ind  = 1 to 4

read eenum ,lcd_char

lcdout I,liner,lcd_char

liner = liner + 1

eenum = eenum + 1

next ind

eenum = 144

liner = 157

for ind  = 1 to 4

read eenum ,lcd_char

lcdout I,liner,lcd_char

liner = liner + 1

eenum = eenum + 1

next ind

eenum = 148

liner = 164

for ind  = 1 to 4

read eenum ,lcd_char

lcdout I,liner,lcd_char

liner = liner + 1

eenum = eenum + 1

next ind

eenum = 152

liner = 214

for ind  = 1 to 4

read eenum ,lcd_char

lcdout I,liner,lcd_char

liner = liner + 1

eenum = eenum + 1

next ind

eenum = 156

liner = 221

for ind  = 1 to 4

read eenum ,lcd_char

lcdout I,liner,lcd_char

liner = liner + 1

eenum = eenum + 1

next ind

pause 300

numlok = 0

keypress = 199

eenum = 128

goto Phone_set

endif

return

Phone_set:

Lcdout I,keypress

gosub inkeys

if Inc_D_Flag = 1 then

lcd_char = key1

if key = 9 then

keydata = 12

flagb = 1

keypress = keypress - 1

eenum = eenum - 1

if eenum = 127 then eenum = 128

pause 100                           '****debounce

endif

if key = 11 then

keydata = 12

flagb = 1

keypress = keypress + 1

eenum = eenum + 1

if eenum = 160 then eenum = 159

pause 100                            '****debounce

endif

if keypress = 198 then keypress = 199

if keypress = 211 then keypress = 150

if keypress = 154 then keypress = 157

if keypress = 161 then keypress = 164

if keypress = 168 then keypress = 214

if keypress = 149 then keypress = 210

if keypress = 156 then keypress = 153

if keypress = 163 then keypress = 160

if keypress = 213 then keypress = 167

if keypress = 220 then keypress = 217

if keypress = 227 then keypress = 224

if keypress = 218 then keypress = 221

if keypress = 225 then keypress = 1

Lcdout I,keypress,lcd_char

if key <> 11 then

if key <> 9 then write eenum ,lcd_char    '******do the eeprom saving stuff here!!!

endif

endif

if keypress = 1 then

Lcdout I,Clr,I,$0C

pause 500

return

endif

goto Phone_set

@    INT_DISABLE   TMR1_INT

'

'	Subroutine Loads TMR1 values

'	============================

SetTimer:

T1CON.0=0			' Stop the Clock

TMR1RunOn.Highbyte=TMR1H	' Load the Run-On (Over-Run) value (if any)

TMR1RunOn.Lowbyte=TMR1L

TMR1RunOn=TMR1Preset+TMR1RunOn	' Calculate the New (adjusted) value for TMR1

If TMR1CalAR=0 then		' Calibration ADVANCE (add) or RETARD (subtract)

TMR1RunOn=TMR1RunOn+TMR1Cal

else

TMR1RunOn=TMR1RunOn-TMR1Cal

endif

TMR1H=TMR1RunOn.Highbyte		' Save new values to TMR1

TMR1L=TMR1RunOn.Lowbyte

T1CON.0=1			' Restart the Clock

PIR1.0=0			' Reset TMR1's Interupt Flag

Return

'

'	Timer Interrupt Handler

'	=======================

TickCount:

'        IF RCIF THEN goto USART_ISR

Gosub SetTimer			' Set the Timer for next 10mS Interrupt

Hundredths=Hundredths+1		' Increment 10mS Seconds Counter

If Hundredths>99 then

Hundredths=0

Seconds=Seconds+1

'    		Gosub Temperature

' Increment the Seconds

If Seconds>59 then

Seconds=0

Minutes=Minutes+1

' Increment the Minutes

If Minutes>59 then

Minutes=0

Hours=Hours+1

If Hours > 23 then    'Increment the Hours

Hours=0

Days = Days+1

If Days>30 then Days=0    'Increment the Hours

endif

endif

endif

endif

@ INT_RETURN

'    	Resume

'    Disable interrupt

@    INT_DISABLE   RX_INT     ; enable USART  interrupts

'@    INT_DISABLE   TMR1_INT

USART_ISR:

HSERIN 1500,goback,[wait ("39") ,STR BytesReply\9]

HSERIN 1000,goback,[wait ("@") ,STR BytesIn\4]

eenum = 140

Value2 = 0

For Value = 1 to 5

Value1 = 0

total = 0

for ind  = 1 to 4

read eenum ,ByteStore[Value1]

BytesPhone[Value2] = ByteStore[Value1]

eenum = eenum + 1

ByteStore =  ByteStore[Value1] - BytesIn[Value1]

if ByteStore = 0  then total = total + 1

Value1 = Value1 + 1

Value2 = Value2 + 1

if total = 4  then

'If ByteStore - BytesIn = 0 then

If eenum = 144 then portb.4 = portb.4^1

If eenum = 148 then portb.3 = portb.3^1

If eenum = 152 then portb.2 = portb.2^1

If eenum = 156 then portb.1 = portb.1^1

If eenum = 160 then portb.0 = portb.0^1

endif

next ind

Next Value

Goto SMS_Transmit

Heat_Transmit:

heat_flag = heat_flag + 1

if heat_flag = 1 then CREN = 0

if heat_flag > 2 then heat_flag = 2

if heat_flag > 1 then Displayloop

SMS_Transmit:

CREN = 0

lcdout I,clr,I,130,"Reception of SMS",I,196,"In-Operation"_

,I,153,"09",STR BytesReply\9,I,217,STR BytesIn\4

for ind  = 0 to 59           '********AT command sending limit each character to 50ms

Lookup ind, ["A","T","+","C","N","M","I","=","1",44,"2",44,"0",44,"0",44,"0",13,10_

,"A","T","E","0",13,10,"A","T","+","C","M","G","F","=","1",13,10,"A","T"_

,"+","C","S","C","A","=",34,"6","3","9","1","8","0","0","0","0","1","0","1"_

,34,13,10],lcd_char

HSerout [lcd_char]

pause 50

next ind

for ind  = 0 to 11           '********cel no. retrieval  limit each character to 50ms

Lookup ind,["A","T","+","C","M","G","S","=",34,"6","3","9"],lcd_char

HSerout [lcd_char]

pause 50

next ind

Value1 = 0

For ind  = 1 to 9                      'cel no. from celfone real time transfer

HSEROUT [STR BytesReply[Value1]]

Value1 = Value1 + 1

pause 50

Next ind

HSerout [34]: pause 50: HSerout [13]: pause 50: HSerout [10]: pause 50

Value1 = 0                                   ' PortB SMS reporting

For Value = 4 to 0  step -1

for ind = 1 to 4

HSEROUT [STR BytesPhone[Value1]]     '," = ",bin1 portb.0

Value1 = Value1 + 1

pause 50

next ind

'                HSEROUT ["="]: pause 50

HSEROUT [" "]: pause 50

if PortB.0(Value) = 1 then

HSEROUT ["O"]: pause 50: HSEROUT ["n"]: pause 50

else

HSEROUT ["O"]: pause 50: HSEROUT ["f"]: pause 50:HSEROUT ["f"]: pause 50

endif

'                HSEROUT [BIN PortB.0(Value)]: pause 50

'                HSEROUT [" "]: pause 50

HSEROUT [13]: pause 50: HSEROUT [10]: pause 50

Next Value

for ind  = 0 to 11           '********cel no. retrieval  limit each character to 50ms

Lookup ind,["T","e","m","p","e","r","a","t","u","r","e"," "],lcd_char

HSerout [lcd_char]

pause 50

next ind

value1 = smoothed DIG 1   : value = smoothed DIG 0

HSEROUT [DEC1 value1]: pause 50: HSEROUT [DEC1 value]: pause 50: HSEROUT ["C"]: pause 50

HSerout [26]: pause 50

goback:

WHILE RCIF            ' Keep reading until RCIF is clear to flush buffer

GP = RCREG             ' after EOM is received

'RCIF = 0

WEND

CREN = 0                        ' Disable continuous receive to clear overrun flag

CREN = 1                        ' Enable continuous receive

'lcdout I,clr,I,Line1,"Multi-Function Timer",I,196,"In-Operation",I,151,"Stand-by Pls!"

'        heat_flag = 0

lcdout I,clr

'        ButtonPress = 1

'       if heat_flag = 1 then DisplayRestart

@ INT_RETURN

End