define osc 4                     'Set oscillator to 4 MHz

define ADC_BITS 10               'Number of bits in result

define ADC_CLOCK 3               'ADC clock source

define ADC_SAMPLEUS 3500         'ADC sampling time = 3.5 ms

define  LCD_DREG        PORTB       ' set LCD Data port

'

define  LCD_DBIT        4           ' set starting data bit

'

define  LCD_RSREG       PORTB       ' set LCD registers

'

define  LCD_RSBIT       2           ' set LCD register select bit

'

define  LCD_EREG        PORTB       ' set LCD enable port

'

define  LCD_EBIT        3           ' set LCD enable bit

'

DEFINE  LCD_BITS        4           ' set LCD bus size

'

define  LCD_LINES       4           ' set number of lines on LCD

'

define  LCD_COMMANDUS   2000        ' set command delay time in us

'

define  LCD_DATAUS      50          ' set data delay time in us

'''''''''''''''''

'''Variables'''

v1     var word                  'Variable where digital value will be stored

'''''''''''''''

'''TRIS registers'''

trisb  = %00000000               'Set PORTB pins as outputs

''''''''''''''''''''

'    This 'INCLUDE' routine is use to read a matrix keypad.

'    Allow to use various keypad format

'    Allow to place the keypad ROW and COL on different PORT.

'    The routine set TRIS register for you

'    How to use it

'    =============

'    1. Include the file

'           write

'               INCLUDE "KeyPad.bas"

'           somewhere in your code.. after TRIS setting and before MAIN loop

'

'    2. Call the Keypad routine by one or the other method bellow

'           1. By calling the macro

'                  @ READKEYPAD _YourVar

'              result store in 'YourVar' wich must be defined as a BYTE sized variable

'

'           2. By using GOSUB

'                  GOSUB KeypadScan

'              result is store in 'Key' Variable

'    Specific DEFINE

'    ===============

'    The defaults setings are

'        1. Keypad ROW are connected to PORTB<3:0>

'        2. Keypad COL are connected to PORTB<7:4>

'        3. Debounce delay = 200 mSec

'        4. No auto-repeat

'        5. Keypad type 4X4

'

'    If you decide to change default settings, here's the DEFINEs list

'        DEFINE KEYPAD_ROW        4        ' 4 ROW keypad

'        DEFINE KEYPAD_ROW_PORT   PORTB    ' ROW port = PORTB

'        DEFINE KEYPAD_ROW_BIT    4        ' ROW0 = PORTB.4

'        DEFINE KEYPAD_COL        4        ' 4 COL keypad

'        DEFINE KEYPAD_COL_PORT   PORTB    ' COL port = PORTB

'        DEFINE KEYPAD_COL_BIT    0        ' COL0 = PORTB.0

'        DEFINE KEYPAD_DEBOUNCEMS 200      ' debounce delay = 200 mSec

'        DEFINE KEYPAD_AUTOREPEAT 1        ' use auto-repeat feature

'

'    When using the auto-repeat feature, the delay is the debounce delay (DEBOUNCEMS)

'    Hardware Connection

'    ===================

'    1. Column bits must have Pull-up resistor attach to VCC

'    2. ROW bits must be connected in serie with resistor.

'    Results

'    =======

'    Results are always set the same way, even with different keypad type

'

'            3X4

'            ---

'                       Col0 Col1 Col2 Col3

'                       ---- ---- ---- ----

'                         1    2    3    4  Row0

'                         5    6    7    8  Row1

'                         9   10   11   12  Row2

'

'            6X5

'            ---

'                       Col0 Col1 Col2 Col3 Col4

'                       ---- ---- ---- ---- ----

'                         1    2    3    4    5  Row0

'                         6    7    8    9   10  Row1

'                        11   12   13   14   15  Row2

'                        16   17   18   19   20  Row3

'                        21   22   23   24   25  Row4

'                        26   27   28   29   30  Row5

'

asm

errorlevel 0,-207                                  ; Disable 'found opcode after...' warning

;

;    Macro Definition

;    ================

READKEYPAD macro KeyVar

L?CALL _KeypadScan

MOVE?BB _Key,KeyVar

endm

;

;    Default settings

;    ================

KB_ROW        = 4                                      ; 4 ROW keypad

KB_ROW_PORT   = PORTC                                  ; Keypad ROW on PORTC

KB_ROW_BIT    = 0                                      ; ROW0 = PORTB.0

KB_COL        = 4                                      ; 4 COL keypad

KB_COL_PORT   = PORTC                                  ; Keypad Col on PORTC

KB_COL_BIT    = 4                                      ; COL0 = PORTB.4

DebounceDelay = d'200'                                 ; debounce delay 200mSec

;

;    Software Definition

;    ===================

;

;    Keypad type definition

;    ----------------------

ifdef KEYPAD_ROW

KB_ROW = KEYPAD_ROW                              ; Assign amount of ROW

endif

ifdef KEYPAD_COL

KB_COL = KEYPAD_COL                              ; Assign amount of COL

endif

;

;    ROW pins assignement

;    --------------------

ifdef KEYPAD_ROW_PORT

KB_ROW_PORT = KEYPAD_ROW_PORT                    ; Assign ROW to user selected PORT

endif

ifdef KEYPAD_ROW_BIT

KB_ROW_BIT = KEYPAD_ROW_BIT                      ; Assign ROW0 to user selected BIT

endif

;

;    COLUMN pins assignement

;    -----------------------

ifdef KEYPAD_COL_PORT

KB_COL_PORT = KEYPAD_COL_PORT                    ; Assign COL to user selected PORT

endif

ifdef KEYPAD_COL_BIT

KB_COL_BIT = KEYPAD_COL_BIT                      ; Assign COL0 to user selected BIT

endif

;

;    Keypad debounce delay

;    ---------------------

ifdef KEYPAD_DEBOUNCEMS

DebounceDelay=KEYPAD_DEBOUNCEMS

endif

endasm

'

'    Variable definition

'    ===================

KB_ROW_PORT         VAR BYTE EXT                       ' ROW port

KB_COL_PORT         VAR BYTE EXT                       ' COL port

Col                 var byte                           '

Row                 var byte                           '

RowBit              var byte                           '

Key                 var byte                           ' result of scanned key

KeyTemp             var byte             '

'

'    Constant definition

'    ===================

KeyMask             con      EXT                       ' Key mask when reading PORT

InitialState        con      EXT                       ' initial ROW PORT value

KB_ROW              CON      EXT                       ' # row

KB_COL              CON      EXT                       ' # column

KB_ROW_BIT          CON      EXT                       ' ROW0 bit

KB_COL_BIT          CON      EXT                       ' COL0 bit

DebounceDelay       CON      EXT                       ' Debounce delay, also Auto-Repeat delay if defined

asm

KeyMask = b'11111111' >> (8-KB_COL)                ; use to keep only COL bits

InitialState=b'00001111' << KB_ROW_BIT             ; Initial state of ROW column bits

endasm

start:

Pause   1500

lcdout  $fe, 1                  ' clear display

lcdout  $fe, $80                ' move cursor to begining of line

lcdout  "     Welcome to JDA Tech "

lcdout  $fe, $C0

Lcdout  "  Wireless Sleep Apnea "       ' output the following

lcdout  $fe, $94

LCDout  "       Monitor"

pause   3000

'

Main:                               ' Operations todo

'

Lcdout  $fe, 1

lcdout  $fe, $80

lcdout  " Choose an Option"

pause  3000

lcdout  $fe,1

'

lcdout  $fe, $80

lcdout  "1 :Start to Record"

lcdout  $fe, $C0

lcdout  "2 :Stop  "          ' display on LCD

pause   10000

'

lcdout  $fe,  1

lcdout  $fe, $80

lcdout  "3: Send Data to RS-232"

lcdout  $fe, $C0

Lcdout  "4: Last Reading Percentage"

gosub   Keypadscan

If (key = 1) then

pause 100

lcdout  $fe, $D4,   #key

Goto Record

endif

IF (key = 2) then

Pause 100

lcdout  $fe, $D4,   #key

Stop

Endif

IF ( key = 3 ) Then

Pause 100

lcdout  $fe, $D4,   #key

GOTO Send

endif

If ( key = 5 ) then

Pause 100

lcdout  $fe, $D4,   #key

Goto Percentage

endif

IF ( key > 5 ) then

pause 100

lcdout $fe, $C0

Lcdout "Invalid Choice"

GOTO main

ENDIF

Record:

GOTO Main

Send:

GOTO Main

Percentage:

Goto Main

goto    JumpOverKeyScan

KeypadScan:

'

'    Set according TRIS registers

'    ============================

asm

variable BitSelect

TRIS_COL = (TRISA + (KB_COL_PORT - PORTA))         ; Find COL TRIS register => input

CHK?RP TRIS_COL                               ; Move to correct bank

BitSelect = 0                                 ;

while BitSelect<KB_COL                        ; Set according TRIS

bsf TRIS_COL, KB_COL_BIT+BitSelect      ;     register bits

BitSelect +=1                           ;     to input

endw

TRIS_ROW = (TRISA + (KB_ROW_PORT - PORTA))         ; Find ROW TRIS register => output

CHK?RP TRIS_ROW                               ; Move to correct bank

BitSelect = 0                                 ;

while BitSelect<KB_ROW                        ; Set according TRIS

bcf TRIS_ROW, KB_ROW_BIT + BitSelect    ;     register bits

BitSelect +=1                           ;     to output

endw

endasm

'

'    Scan the Keypad

'    ===============

repeat

key=0                                              ' no key is press

row=0                                              ' begin scan @ ROW0

col=0                                              '              COL0

kb_row_PORT=kb_row_PORT | InitialState             ' set all ROW bits to 1

repeat

ROWBIT=ROW+KB_ROW_BIT                          ' point to a ROW bit

kb_row_PORT.0[RowBit]=0                        ' clear actual ROW bit

Gosub readcol                                  ' Read  column

if KeyTemp != KeyMask then                     ' Any column to 0?

REPEAT                                      '    YES

IF KeyTemp.0=0 THEN                      '       IF the actual column=0

key=row*KB_COL                       '          Calculate

key=key+col                          '              the

key=key+1                            '                  Value

row=KB_ROW                           '       Set maximum ROW to getout of here

@ ifndef KEYPAD_AUTOREPEAT           '       If AutoRepeat is not DEFINE

repeat

Gosub readcol           '           Wait until

until KeyTemp=keymask   '               Key is released

@ endif

PAUSE DebounceDelay                  '        Debounce delay

ENDIF

keytemp=keytemp>>1                       ' shift to next column bit

Col=col+1                                ' go to next column

UNTIL COL=KB_COL                         ' COL scan Finished?

endif

kb_row_PORT.0[Rowbit]=1                        ' set actual ROW bit

row=row+1                                      ' point to next row

until row>(KB_ROW-1)

until key!=0                                       ' Row scan finished?

RETURN                                                 ' any key pressed?

ReadCol:

KeyTemp=kb_col_PORT>>kb_col_Bit                        ' Read Column state

KeyTemp=KeyTemp & kEYMASK                              ' Keep only wanted bits

return

JumpOverKeyScan: