/* Project name: DC-AC Inverter

* Author: BABALOLA

* Description:

This project generate sine-wave PWM in an open loop

using timer0 to generate interupt.

* Test configuration:

MCU: P18F452

Oscillator: HS, 20.0000 MHz

SW: mikroC v8.2

24/03/2010

* NOTES:

*/

unsigned char const sine_wave[32]= {20,41,60,80,98,116,132,147,161,173,183,

192,199,204,207,208,207,204,199,192,183,173,161,147,132,116,98,80,60,41,20,0};

//unsigned char const sine_wave[32]={20,40,59,78,96,113,129,144,158,170,180,188,

//195,200,203,204,203,200,195,188,180,170,158,144,129,113,96,78,59,40,20,0};

//unsigned int const sine_wave[32]= {37,74,110,145,179,211,241,269,294,316,335,351,364,

//373,378,380,378,373,364,351,335,316,294,269,241,211,179,145,110,74,37,0};

// variable/function definitions

unsigned char index;

int duty;

// TIMER INTERRUPT SERVIVE ROUTINE

//Inverter run in close-loop mode

void Interrupt( )

{

INTCON.TMR0IF = 0;     // clear TMR0IF(interupt flag that brought us here)

//T0CON.TMR0ON = 0;    // STOP TIMER0

TMR0H = 0xF9;          // Reload Timer0 register

TMR0L = 0xE5;

PORTC.F0 =0;   //enable inverter

CCPR1L = sine_wave[index];

if (!index)

PORTC.F1 = ~PORTC.F1;

++index;

if (index == 32)

index = 0;

}

void main( )

{

//Prepare PIC for operation,intialise variables and the controller hardware

INTCON.F7 = 0;                //diable all interrupt now

TRISC = 0X90;                   //set  portc direction

PORTC.F0 = 1;                   //disable inverter driver

TRISB = 0XC9;                   //portb direction

//PORTB.F1 = 1;                   //hold power supply

PORTB.F4 = 1;                   // TEST LEDS

PORTB.F5 = 0;

delay_ms(1000);

PORTB.F4 = 0;

PORTB.F5 = 1;

delay_ms(1000);

PORTB.F5 = 0;

index = 0;

// pwm setup

PR2 = 0XFF;                      // 20 MHz clock - > 19.53kHz PWM frequency

T2CON =0X04;                     // timer2 on  , NO pre/postscale

CCPR1L = 0;                      // Initial Duty

CCP1CON =0X0C;                   // configure and start as pwm

// Timer0 Setup

T0CON = 0X08;                  // Timer0 settings,no presc,timer start now

TMR0H = 0xF9;

TMR0L = 0xE5;               // Initialize Timer0 register

//set up the interrupts, enable it and start timer0

INTCON =0X20;                   //SETUP GLOBAL INT.

T0CON.F7  = 1;           // enable Timer0 interrupt

INTCON.F7 = 1;                  //enable global interrupt

PORTB.F4 = 1;                   //GET READY (green led on)

while( 1 );                     //loop forever, execute in INTERRUPT

}

Below is my basic schematic with information about the transformer. My output target is 500WATT.

INFORMATION ON TRANSFORMER:

RATIO IS 1:17

LEAKAGE INDUCTANCE IS 2.76mh which is used with the output capacitor to form the output filter.

With the accompanied code, I’ am getting 246vac without load at 24 battery voltage.

REGARDS,

BABALOLA.