'

@ device pic16F716,xt_osc,wdt_off,pwrt_on,protect_off

'

'

'************Define Display Characters***********************************

ZERO	CON		$7E		'0-7 seg. code for zero

ONE		CON		$48

TWO		CON		$3D

THREE	CON		$6D

FOUR	CON		$4B

FIVE 	CON		$67

SIX		CON		$77

SEVEN	CON		$4C

EIGHT 	CON		$7F

NINE 	CON		$6F

BLANK 	CON		$00

DISPL 	CON		%00110010	'L

DASH 	CON		%00000001	' -

AONE	CON		%00001000	' 1 ON LCDA

PLUS	CON		%1000000	'  + set BIT 7 & 6 of LCDA

MINUS 	CON		%01000000	' - set BIT 6 of LCDA

DP1		CON		%10000000	' dp1 set BIT 7 of LCDD

DP2		CON		%00000100	' dp2 set BIT 2 of LCDA

DP3		CON		%00100000	' dp3 set BIT 6 of LCDA

BLANKDP3 CON		%11011111	' dp3 Clear BIT 6 of LCDA

o		CON		%01110001	' small o

n		CON		%01010001	' small n

LO		CON		$7E			' large O (same as ZERO)

LF		CON		%00010111	' large F

LE		CON		%00110111	' large E

LC		CON		%00110110	' large C

BLANKONE CON	%11110111	' blank 1 ON A

LP		CON		%00011111	' large P

LA		CON		%01011111	' large A

'

'

'*************************************************************************

'

'************Define Variables*********************************************

'

adval   VAR BYTE

b1      VAR BYTE

b3      VAR BYTE

b5      VAR BYTE

c0		VAR BYTE

c1		VAR BYTE

c2		VAR BYTE

c3		VAR BYTE

y		VAR	BYTE

x		VAR	BYTE

time    VAR BYTE

longt   VAR BYTE

lx		VAR	WORD

'

'**************************************************************************

'

'

wsave      	VAR     	BYTE $020 SYSTEM

ssave      	VAR     	BYTE	BANK0 SYSTEM

psave      	VAR 		BYTE BANK0 SYSTEM

NewValue   	VAR     	BYTE           'store serial data here

LCDA		VAR			BYTE

LCDB		VAR			BYTE

LCDC		VAR			BYTE

LCDD		VAR			BYTE

L_CHAR		VAR			BYTE

LCD_CLK		VAR			PORTB.7

LCD_SEL		VAR			PORTB.5

LCD_DOUT	VAR			PORTB.6

'

'

GoTo start                        'jump around asm interrupt handler

'

Asm

table

ADDWF  PCL,1           	;add value to prog. count

RETLW  0x7e           	;and return code to w

RETLW  0x48

RETLW  0x3D

RETLW  0x6D

RETLW  0x4B

RETLW  0x67

RETLW  0x77

RETLW  0x4C

RETLW  0x7F

RETLW  0x6F

EndAsm

'

'************Main PIC BASIC Program Begins here****************************

'

Start:

'

'*********Initalize ports***************************************************

'

TRISA = %00000         'Set as Ouptuts

TRISB = %00000000      'Set as Outputs

'

'adcon1 = 0            'set RA0-RA3 as analog inputs

'

'****************************************************************************

'

DEFINE ADC_BITS    8         'set number of bits in results

DEFINE ADC_CLOCK   3         'set clock source (3=rc)

DEFINE ADC_SAMPLEUS    50    'set sampling time in us

'

'****************************************************************************

'

Main:

'

LCDA = Blank               'clear display at the start

LCDb = blank

lcdc = blank

lcdd = blank

'

B1 = 1

'

Normal:

lx = b1

Call display

b1 = b1+1

pause 250

if b1 = 101 then main

goto normal

display:

'****************************************************

' This subroutine writes the values contained in

' LCD_A, LCD_B, LCD_C, AND LCD_D TO the display

' *****************************************************/

Call convertdigits

LCD_CLK = 0			' Clear clock

LCD_DOUT = 0

' NOTE!! Display first BYTE last TO compensate For mis-wired display board

Asm

movf	_LCDB,0

Call	table

movwf	_L_CHAR		;LSD+2

Call	_wdisplay

movf	_LCDC,0

Call	table

movwf	_L_CHAR 	;LSD+1

Call 	_wdisplay

movf	_LCDD,0

Call	table

movwf	_L_CHAR		;LSD Digit 1 (LSD+0)

Call 	_wdisplay

movf	_LCDA,0			;MSD Digit 4 (LSD+3)

;Call	table

movwf	_L_CHAR

Call 	_wdisplay

EndAsm

LCD_SEL = 1			' loads data in LCD

LCD_SEL = 0

Return

' ************* Subroutine - Write display**********

wdisplay:

For y = 0 TO 7		' bit counter

Asm

rrf		_L_CHAR,1	; rotate LSB into carry

btfsc	status,0

bsf		PORTB.6

EndAsm

'Pause(1)

LCD_CLK = 1		' clock in data

'Pause(1)

LCD_CLK = 0

'Pause(1)

LCD_DOUT = 0

Next y

Return

'

convertdigits:

x = 0				' 100's digit

LCDB = 0			' 10's digit

LCDC = 0 			' 1's digit

LCDD = 0			'  .1's digit

While lx >= 1000

x=x+1

lx = lx - 1000

Wend

While (lx >= 100)

LCDB = LCDB+1

lx = lx - 100

Wend

While (lx >= 10)

LCDC = LCDC+1

lx = lx - 10

Wend

LCDD = lx				' .1's digit - convert back to 8 bits

IF x = 0 Then

LCDA = LCDA & BLANKONE

Else

LCDA = LCDA | AONE;			' set 1

EndIF

Return

End