'****************************************************************

'*  Name    : Randomize 20 LEDs.BAS                             *

'*  Author  : Greg Browne                                       *

'*  Notice  : Copyright (c) 2011 Not a chance                   *

'*          : All Rights Reserved                               *

'*  Date    : 08/30/2011                                        *

'*  Version : 1.0                                               *

'*  Notes   : 12f683 w/ nap to conserve power and various       *

'*          : lighting sequences                                *

'****************************************************************

' needs to be streamlined for cleaner reading

clear

define osc 4

' ------------------------------------turn off some stuff to save a little power

@ device mclr_off 'start button is connected here

@ device bod_off

@ device fcmen_off

@ device ieso_off

@ device intrc_osc_noclkout

@ device pwrt_on

@ device protect_off

ansel = 0

cmcon0 = 7

adcon0 = 0

' --------------------------------------------------------------- some variables

delay2 var word

myword var word

delay var byte

mybyte var byte

previous_random var byte

gpiostate var byte

tristate var byte

i var byte

j var byte

k var byte

l var byte

' ------------------------------------------------------------------ some values

delay = 50

delay2 = 500

'------------------------------------------------------------finally the program

starthere:

if gpio.3 = 1 then goto main

nap 4   'naps for 288 milliseconds

goto starthere

'--------------------------------------------flash them all to make it look cool

main:

for i = 1 to 20

random myword

mybyte = (myword//20)

if previous_random = mybyte then goto main

gosub high_led

pause delay

gpio = %000000

pause delay

previous_random = mybyte

next i

'---------------------------------------------------------------- first 13 boxes

mybyte = 0' start LEDs in sequence

for mybyte = 0 to 12

gosub high_led

pause delay2*2

gpio=%000000

pause delay

next mybyte

'---------------------------------------------------------- four stars real fast

mybyte = 13' beginning of stars

for j = 1 to 4

for mybyte = 13 to 16

gosub high_led

pause delay2/5

gpio=%000000

pause delay

next mybyte

next j

'---------------------------------------- four stars with another blinking twice

mybyte = 13' beginning of stars

for mybyte = 13 to 17 ' with the HUD glass this time

gosub high_led

pause delay2

gpio=%000000

pause delay

next mybyte

' ---------------------------------------blinks these lights k number of times

mybyte = 17

for k= 1 to 3

gosub high_led

pause 500

gpio = 0

pause 100

next k

'-----------------------------------now for the flag and name//take this out ???

'        mybyte = 18' show them off a little

'for mybyte = 18 to 19

'        gosub high_led

'        pause delay2

'        gpio=%000000

'        pause delay

'next mybyte

'----------------------------------------light american flag first then name tag

trisio = %001111

gpio = 0

for l = 0 to 255 step 5' led18 gpio.4

pwm gpio.4,l,10' light number 18, l intensity step, 10 usec

next l

for l = 255 to 0 step -5

pwm gpio.4,l,10

next l

trisio = %001111

for l = 0 to 255 step 5 'led 19 is gpio.5

pwm gpio.5,l,10' light number 19, l intensity step, 10 usec

next l

for l = 255 to 0 step -5

pwm gpio.5,l,10

next l

'----------------------------------------------------------------turn it all off

gpio = 0

trisio = %111111

' -----------------------------------------------------------send it back to nap

goto starthere

high_led:

lookup mybyte,[%000001,%000010,%000001,%000100,%000001,%010000,%000001,%100000,%000010,%000100,%000010,%010000,%000010,%100000,%000100,%010000,%000100,%100000,%010000,%100000],gpiostate

gpio = gpiostate

lookup mybyte,[%111100,%111100,%111010,%111010,%101110,%101110,%011110,%011110,%111001,%111001,%101101,%101101,%011101,%011101,%101011,%101011,%011011,%011011,%001111,%001111],tristate

trisio = tristate

return

end