'****************************************************************

'*  Name    : Randomize 20 LEDs.BAS                             *

'*  Author  : Greg Browne                                       *

'*  Notice  : Copyright (c) 2011 Not a chance                   *

'*          : All Rights Reserved                               *

'*  Date    : 08/30/2011                                        *

'*  Version : 1.0                                               *

'*  Notes   : 12f683, w/ nap to conserve power and various      *

'*          : lighting sequences                                *

'****************************************************************

CLEAR

DEFINE osc 4

' ------------------------------------turn off some stuff to save a little power

@ device mclr_off 'start button is connected here

@ device bod_off

@ device fcmen_off

@ device ieso_off

@ device intrc_osc_noclkout

@ device pwrt_on

@ device protect_off

ansel = 0

cmcon0 = 7

adcon0 = 0

' --------------------------------------------------------------- some variables

delay2 VAR WORD

myword VAR WORD

delay VAR BYTE

mybyte VAR BYTE

previous_random VAR BYTE

gpiostate VAR BYTE

tristate VAR BYTE

i VAR BYTE

j VAR BYTE

k VAR BYTE

l VAR BYTE

' ------------------------------------------------------------------ some values

delay = 50

delay2 = 500

'------------------------------------------------------------finally the program

starthere:

IF gpio.3 = 1 THEN GOTO main

NAP 4   'naps for 288 milliseconds

GOTO starthere

'--------------------------------------------flash them all to make it look cool

main:

FOR i = 1 TO 20

RANDOM myword

mybyte = (myword//20)

IF previous_random = mybyte THEN GOTO main

GOSUB high_led

PAUSE delay

gpio = %000000

PAUSE delay

previous_random = mybyte

NEXT i

'---------------------------------------------------------------- first 13 boxes

mybyte = 0' start LEDs in sequence

FOR mybyte = 0 TO 12

GOSUB high_led

PAUSE delay2*2

gpio=%000000

PAUSE delay

NEXT mybyte

'---------------------------------------------------------- four star LEDs real fast

mybyte = 13' beginning of stars

FOR j = 1 TO 4

FOR mybyte = 13 TO 16

GOSUB high_led

PAUSE delay2/5

gpio=%000000

PAUSE delay

NEXT mybyte

NEXT j

'---------------------------------------- four LEDs with another blinking twice

mybyte = 13' beginning of stars

FOR mybyte = 13 TO 17 ' with the HUD glass this time

GOSUB high_led

PAUSE delay2

gpio=%000000

PAUSE delay

NEXT mybyte

' ---------------------------------------blinks the LED17 k number of times

mybyte = 17

FOR k= 1 TO 3

GOSUB high_led

PAUSE 500

gpio = 0

PAUSE 100

NEXT k

'----------------------------------------light american flag first then name tag

trisio = %001111

gpio = 0

FOR l = 0 TO 255 STEP 5' led18 gpio.4

PWM gpio.4,l,10' light number 18, l intensity step, 10 usec

NEXT l

FOR l = 255 TO 0 STEP -5

PWM gpio.4,l,10

NEXT l

trisio = %001111

FOR l = 0 TO 255 STEP 5 'led 19 is gpio.5

PWM gpio.5,l,10' light number 19, l intensity step, 10 usec

NEXT l

FOR l = 255 TO 0 STEP -5

PWM gpio.5,l,10

NEXT l

'----------------------------------------------------------------turn it all off

gpio = 0

trisio = %111111

' -----------------------------------------------------------send it back to nap

GOTO starthere

high_led:

LOOKUP mybyte,[%000001,%000010,%000001,%000100,%000001,%010000,%000001,%100000,%000010,%000100,%000010,%010000,%000010,%100000,%000100,%010000,%000100,%100000,%010000,%100000],gpiostate

gpio = gpiostate

LOOKUP mybyte,[%111100,%111100,%111010,%111010,%101110,%101110,%011110,%011110,%111001,%111001,%101101,%101101,%011101,%011101,%101011,%101011,%011011,%011011,%001111,%001111],tristate

trisio = tristate

RETURN

END