Include "Modedefs.bas"

' ** Set Xtal Value in mHz **

Define	OSC	20		' Set Xtal Frequency

#CONFIG

__config _HS_OSC & _WDT_ON & _MCLRE_ON & _LVP_OFF & _CP_OFF

#ENDCONFIG

' ** Declare Pins Used **

CMCON= 7

INPUT	    PortA.0		' Data is clocked on rising edge of this pin

' ** Variables **

xRaw      VAR   Word   ' Pulse width from Memsic 2125

xxRaw    VAR   Word

xxxRaw  VAR   Word

xGForce VAR   Word    ' x axis g force (1000ths)

xTilt       VAR   Word    ' x axis tilt (100ths)

idx          VAR   Byte     ' Table index

mult        VAR   Word   ' Multiplier - whole part

frac         VAR   Word    ' Multiplier - fractional part

HiPulse   con 1               ' Tells pulsin to look for highpulse, not lowpulse

Read_X_Force:

PULSIN PORTA.0, HiPulse, xRaw

pause 1000

xRaw = xRaw * 20

'   xGForce = ((xRaw / 99) - 502) / 8   ' g = ((t1 / 0.01) - 0.5) / 12.5%  formula from data sheet Modified for our PIC - Converts to 1/1000 g's

xxRaw = xraw /99

xxxRaw = xxRaw -502

xGForce = xxxRaw /8

Read_X_Tilt:

'tilt = g x k Select tilt conversion factor based on static G force.

'Table data derived from Memsic specs.

LOOKDOWN2 ABS xGForce, <=[174, 344, 508, 661, 2000], idx  ' Table is in thousandths g results for 10,20,30,40 and max value degrees

LOOKUP idx, [57, 58, 59, 60, 62], mult          'integer lookup

LOOKUP2 idx, [32768, 10486, 2621, 30802, 22938], frac 'gets fractional result

' G Force is divided by 10 to prevent roll-over errors at end of range. Tilt is returned in 100ths degrees

'Fractional lookup like this: 65,536 * .16 (this is from the fractional part of 20 degree, 58.16 number) = 10486

xTilt = mult * (ABS xGForce / 10) + (frac ** (ABS xGForce / 10))

Check_SignX:

IF (xGForce.Bit15 = 0) THEN XT_Exit ' if positive, skip correct for g force sign

xTilt = -xTilt

XT_Exit:

GOSUB Read_X_Force