'

@  device PIC16F628A

@  device INTRC_OSC_NOCLKOUT, MCLR_OFF

Define  osc             		4       ' Default 4 MHz osc

Define  LCD_DREG        	PORTA   ' Define LCD 'D' connection

Define  LCD_DBIT        	0

Define  LCD_RSREG       	PORTA   ' Define LCD 'RS' connection

Define  LCD_RSBIT      	 4

Define  LCD_EREG        	PORTB   ' Define LCD 'E' connection

Define  LCD_EBIT        	0

' ** Define LCD Control Constants For a 2x40 Display **

CMD      	CON 	$FE     ' Control Byte

Clr      		CON 	$01     ' Clear the display

CrsrHm   	CON 	$02     ' Cursor home

Line1    	CON 	$02     ' Point to beginning of line 1

Line2    	CON 	$C0     ' Point to beginning of line 2

ResetCnt    	VAR PORTB.1 	' To reset count of internal counter in EEPROM

Count_Out  	VAR PORTB.2 	' Count out to driver for counter and LED

PWM_OUT     VAR PORTB.3 	' PWM output to drive Emitter LED

DET_IN      	VAR PORTB.4 	' To input of PIC chip for detection of IR signal

LED1_Audio  	VAR PORTB.5 	' Output to drive PIEZO alarm circuit

SCLK        	VAR PORTB.6 	' Data Pin on I2C

SDATA       	VAR PORTB.7 	' Clock Pin on I2C

addr        	VAR WORD    	' Memory location in EEPROM to use

CntRec      	VAR WORD    	' Value to store in EEPROM

addr    	= $02       		' Location of memory to use

CMCON  	 = 7        		' PORTA digital

VRCON  	 = 0         		' A/D Voltage reference disabled

TRISB  	 = %11010010 	' B.0,B.2,B.3(PWM),B.5 = Outputs

PR2    	 = 25        		' Set PR2 for 38.4kHz PWM [26uS period]

CCPR1L  	= 13        		' Set PWM Duty-Cycle to 50% approx (5 = 20%)

CCP1CON 	= %00001100 		' Mode select = PWM

T2CON  	 = %00000100 	' Timer2 ON + 1:1 prescale

' -------------------------------------------------------------

Pause 200           			' Wait .5 ms for LCD to startup

Lcdout CMD,Clr      		' Clear LCD Display

Pause 100           			' wait 0.1 seconds LCD settle

EEProm_Read:

' /////////////////////////////////////////////////////////////

I2CREAD SDATA,SCLK,$A0,addr,[CntRec]    ' External EEPROM 24LC256

'    Read Loc1, CntRec                      ' Internal EEPROM Pic16F628A

' /////////////////////////////////////////////////////////////

Select Case CntRec

Case 0,255     			' Check if new EEPROM and send message to LCD Display

CntRec = 0

GOSUB LCD_Display03     ' EEPROM is EMPTY

End Select

If ResetCnt = 0 Then

CntRec = 0

GOSUB LCD_Display04         ' Reset Count

Endif

' -------------------------------------------------------------

'

Main:

If (DET_IN = 1) THEN

GOSUB LCD_Display01

' LED1_AUDIO (LED / AUDIO)

LED1_Audio = 1

Pause 1000      			' Pause 1 second

LED1_Audio = 0

' COUNT_OUT (COUNTER)

COUNT_OUT = 1

PAUSE 200       			' wait 0.2 seconds

COUNT_OUT = 0

CntRec = CntRec + 1     		' Increment counter in EEPROM

IF (CntRec > 32766) THEN

CntRec = 1

GOSUB LCD_Display04   	' Reset Count

ENDIF

EEProm_Write:

' /////////////////////////////////////////////////////////////

I2CWRITE SDATA,SCLK,$A0,addr,[CntRec] ' External EEPROM 24LC256

'        WRITE Loc1,CntRec              ' Internal EEPROM Pic16F628A

' /////////////////////////////////////////////////////////////

ELSE

GOSUB LCD_Display02

Endif

GOTO Main

' -------------------------------------------------------------

LCD_Display01:

Lcdout CMD, Line1, "* * * * * * Detected IR Signal * * * * *"

LCDOUT CMD, Line2, "            Detect Total = " ,#CntRec, "           "

RETURN

LCD_Display02:

Lcdout CMD, Line1, "               No Signal                "

LCDOUT CMD, Line2, "            Detect Total = " ,#CntRec, "           "

RETURN

LCD_Display03:

LCDOUT CMD, Line1, "* * * * * * EEPROM is EMPTY * * * * * *"

LCDOUT CMD, Line2, "            EEPROM = " ,#CntRec, "                 "

PAUSE 5000      ' pause for 5 seconds

RETURN

LCD_Display04:

LCDOUT CMD, Line1, "* * * * * * * Reset Count * * * * * * *"

LCDOUT CMD, Line2, "              EEPROM = " ,#CntRec, "               "

PAUSE 5000      ' pause for 5 seconds

RETURN

EndProgram:

End