|  | Javascript Electronic Notebook 868 MHz Yagi Design Details (Metric) 11 Elements, 12.3 dBd Estimated Gain by Martin E. Meserve |
| --- | --- |

| Design Synopsys 868 MHz, 11 Elements, 12.293 dBd Estimated Gain 35.5 Degrees Horizontal Beam Width 37.3 Degrees Vertical Beam Width N/A Diameter, Non-Metalic Boom with Insulated Elements. N0 Boom Correction applied. Electrical Boom Length of 869.7 mm. Allow for overhang when cutting boom to length. 3.000 mm Driven Element Diameter. 3.000 mm Parasitic Element Diameter. Suggested Stacking Distance for 2 Yagis: 495.7 mm Horizontally 471.8 mm Vertically 1.0 mm Dimensional tolerance required for element lengths. | Antenna Dimensions Cumulative Spacing (mm); Element; Element Length (mm). Zero; REFL; 167.73. 69.08; D.E.; 164.51. 94.98; D1; 148.2. 157.15; D2; 146.13. 231.41; D3; 144.12. 317.75; D4; 142.26. 414.46; D5; 140.63. 518.08; D6; 139.22. 626.87; D7; 137.99. 740.85; D8; 136.92. 860.01; D9; 135.97. |
| --- | --- |