| sample voltage divider shown to the right. If the input is 25Vdc then the output voltage will be about 4.38 Vdc. With 10-bit A/D you should get an ADCin of around 898. If you first scale that A/D value up to match the voltage divider ratio, you can then calculate the voltage the same way you would if you were reading 0-5 V. The ratio of the voltage divider is (R1+R2)/R2. To scale up the A/D reading you just multiply the value times the total resistance of the divider (R1+R2) then divide that number by R2. 898 * 5700 / 1000 = 5118 This is what the A/D would be if it could actually read 0-25 volts directly. Now then, for 1 decimal place, multiply that by 50 and divide by 1023. For 2 decimal places, multiply by 500 instead. 5118 * 50 / 1023 = 250 or 25.0Vdc The program below is one way to do it in PBP. HTH, Darrel |  |
| --- | --- |

Code:

Res1   Var Word

Res2   Var Word

Rt     Var Word

Volts  Var Word

AD     Var Word

Loop:

ADCin  0, AD

Res1 = 4700          ' Change these to match your Voltage divider

Res2 = 1000          ' resistor values for the Solar Cell

Gosub CalcVoltage

LCDout $FE,2,"Solar= ",DEC Volts/10,".",DEC1 Volts Dig 0," Vdc"

ADCin  1, AD

Res1 = 2200          ' Change these to match your Voltage divider

Res2 = 1000          ' resistor values for the Battery

Gosub CalcVoltage

LCDout $FE,$C0,"Batt = ",DEC Volts/10,".",DEC1 Volts Dig 0," Vdc"

goto Loop

CalcVoltage:

Rt = Res1 + Res2        ' Total resistance of Voltage Divider

Volts = AD * Rt         ' Scale the AD reading accordingly

Volts = DIV32 Res2

Volts = Volts * 50      ' Convert AD to Voltage

Volts = DIV32 1023

Return