'****************************************************************

'*  Name    : Overlay.BAS                                       *

'*  Author  : [Grant Taylor VE3XTV / ZL1WTT]                    *

'*  Notice  : Copyright (c) 2010                                *

'*          : All Rights Reserved                               *

'*  Date    : Start date 30/12/2010                             *

'*  Version : 6.0                                               *

'*  Notes   : onging project                                    *

'*          :                                                   *

'****************************************************************

' Using STV5730A as a video overlay for text information

' With a PIC16F876A or the PIC18F252 Microcontroller

' using a real time clock PCF8563

' Definitions

DEFINE OSC 20                ' 20 MHz Oscillator frequency

DEFINE HSER_RCSTA       90h  ' Set receive register to receiver enabled

DEFINE HSER_TXSTA       20h  ' Set transmit register to transmitter enabled

DEFINE HSER_BAUD        9600 ' Set baud rate

Define ADC_BITS	10	         ' Set number of bits in result

Define ADC_CLOCK 3	         ' Set clock source (3=rc)

Define ADC_SAMPLEUS 50       ' Set sampling time in uS

'Port Definition

Dpin  var PORTB.0  ' I2C Data pin

Cpin  var PORTB.1  ' I2C Clock pin

DA    VAR PORTB.2  ' Data out

CL    VAR PORTB.3  ' Clock out

EN    VAR PORTB.4  ' Chip enable or chip select

'Variables

RC    VAR BYTE      ' Row count

CC    VAR BYTE      ' Character counter

LC    VAR BYTE      ' Line counter

wData VAR WORD      ' Write data

Echo  var byte      ' Storage for serial character

TAB   VAR BYTE      ' TAB to move across a line

TP    VAR WORD      ' Temp variable holder

T     VAR BYTE      ' Time variable holder

T1    VAR BYTE      ' Time variable holder

T10   VAR BYTE      ' Time variable holder

NC    VAR BYTE      ' Number count

J     VAR WORD      ' Table counter variable

C     VAR BYTE      ' Counter variable

Co    VAR BYTE      ' Counter out variable

L     VAR WORD      ' Line counter

Sz    VAR BYTE      ' Text size

H     VAR BYTE      ' Hex number

AS    VAR BYTE      ' ASCII number

D     VAR BYTE [28] ' ASCII Digits

CS    var byte [7]  ' Callsgin position array

Cx    VAR BIT       ' Date bit

Tx    VAR BIT       ' Date bit

SumA  VAR BYTE      ' Check output pins

SumB  VAR BYTE      ' Check output pins

AD0   VAR WORD	    ' Create ad0 to store result

AD1   VAR WORD	    ' Create ad1 to store result

X     VAR BIT       ' Callsign set bit

Init:

TRISA = %000011          ' Set as both

ADCON1 = %00000010	 ' Set PORTA analog and LEFT justify result

TRISB = %00000011        ' Set as both

TRISC = %11111111        ' Set as both

H=$30 : Tx=1 : Cx=0 : X=0

' STV5730A initialize

initSTV:

High Cl :High DA :High EN       ' Start up data

Pause 250                       ' 250ms wait

wData=$3000 : GoSub Send        ' Send 3000 in Hex

wData=$3000 : GoSub Send        ' Send 3000 in Hex

wData=$00DB : GoSub Send        ' Send 00DB in Hex

wData=$1000 : GoSub Send        ' Send 1000 in Hex

wData=$00CC : GoSub Send        ' Init registers (point to register 12)

wData=$1000 : GoSub Send        ' Zoom

wData=$1000 : GoSub Send        ' Colour

wData=$1ACE : GoSub Send        ' Control set to NTSC

wData=$15A1 : GoSub Send        ' Position

wData=$183C : GoSub Send        ' Mode

wData=$00C0 : GoSub Send        ' Set row attributes

For RC=0 TO 10                  ' Rows from 0 to 10

wData=$10C4 : GoSub Send

Next RC

CC=0: LC=0                      ' Set line and row to 0

GOSUB CLS                       ' Clear video screen

PAUSE 250

CC=0: LC=0                      ' Set line and row to 0

GOSUB CLS                       ' Clear video screen

GOTO com_set

CLS:

wData=lc*256+CC    ' Line and row data

GoSub Send         ' Send data to STV5730A

wData=$1F0B        ' Set character to " "

GoSub Send         ' Send data to STV5730A

CC=CC+1

IF CC>=28 Then     ' Max character count of 28

CC=0

lc=LC+1

EndIF

IF Lc=11 THEN      ' Max line count of 11

RETURN

ELSE

GoTo CLS

ENDIF

Send:         ' Send data to STV5730A

PAUSEUS 250   ' Deylay of 250 uS

Low EN        ' Set chip select low

GoSub Serial  ' Send 16 bit word

High EN       ' Set chip select pin high

High Cl       ' Set clock pin high

High DA       ' Set data pin high

Return

Serial:        ' Serial out routine

DA=wData.15    ' Bring Do pin high or low, depending upon bit

PULSOUT CL,1   ' Bring Clock line high and then low

DA=wData.14    ' Same as above

PULSOUT CL,1   ' Same as above

DA=wData.13

PULSOUT CL,1

DA=wData.12

PULSOUT CL,1

DA=wData.11

PULSOUT CL,1

DA=wData.10

PULSOUT CL,1

DA=wData.9

PULSOUT CL,1

DA=wData.8

PULSOUT CL,1

DA=wData.7

PULSOUT CL,1

DA=wData.6

PULSOUT CL,1

DA=wData.5

PULSOUT CL,1

DA=wData.4

PULSOUT CL,1

DA=wData.3

PULSOUT CL,1

DA=wData.2

PULSOUT CL,1

DA=wData.1

PULSOUT CL,1

DA=wData.0

PULSOUT CL,1

Return

PC_time:                                      ' Time comport

T=0

I2CREAD Dpin,Cpin,$A3,$02,[T]

GOSUB Tcon

CS[0]=T

I2CREAD Dpin,Cpin,$A3,$03,[T]

GOSUB Tcon

CS[1]=T

I2CREAD Dpin,Cpin,$A3,$04,[T]

T=T&$3F

GOSUB Tcon

CS[2]=T

hserout ["Time in h,m,s"]

GOSUB linefeed                                   ' Send text followed by carriage return and linefeed

GOSUB linefeed                                   ' Carriage return and linefeed

Hserout [DEC CS[2],":",DEC CS[1],".",DEC CS[0]]	 ' Send text followed by carriage return and linefeed

GOSUB linefeed

GOSUB linefeed                                   ' Carriage return and linefeed

RETURN

PC_date:                                         ' Date comport

I2CREAD Dpin,Cpin,$A3,$05,[T]

T=T&$3F

GOSUB Tcon

CS[3]=T

I2CREAD Dpin,Cpin,$A3,$06,[T]

T=T&$07

GOSUB Tcon

CS[4]=T

I2CREAD Dpin,Cpin,$A3,$07,[T]

T=T&$1F

GOSUB Tcon

CS[5]=T

I2CREAD Dpin,Cpin,$A3,$08,[T]

GOSUB Tcon

CS[6]=T

hserout ["Date in d,m,y"]                  ' Send text followed by carriage return and linefeed

GOSUB linefeed

GOSUB linefeed                                   ' Carriage return and linefeed

IF CS[4]=0 THEN hserout ["Sunday",13,10]

IF CS[4]=1 THEN hserout ["Monday",13,10]

IF CS[4]=2 THEN hserout ["Tuesday",13,10]

IF CS[4]=3 THEN hserout ["Wednesday",13,10]

IF CS[4]=4 THEN hserout ["Thursday",13,10]

IF CS[4]=5 THEN hserout ["Friday",13,10]

IF CS[4]=6 THEN hserout ["Saturday",13,10]

GOSUB linefeed                                    ' Carriage return and linefeed

Hserout [DEC CS[3],"/",DEC CS[5],"/20",DEC CS[6]] ' Send text followed by carriage return and linefeed

GOSUB linefeed

GOSUB linefeed                                    ' Carriage return and linefeed

RETURN

Tcon:

T=T&$7F

T1 =($0F&T)

T10=(T/$10)*10

T=T10+T1

RETURN

Set_time:

T10=(T10-H)*$10

T1 =T1-H

Co=T10+T1

RETURN

vcon:

CS[1]=T DIG 0     ' Read the position of digit 2

CS[0]=T DIG 1     ' Read the position of digit 1

RETURN

v_time:           ' Display time via the video

J=J+1

I2CREAD Dpin,Cpin,$A3,$05,[T]

T=T&$3F

SumB=t

IF Cx=1 AND SumA<>SumB THEN GOSUB v_date

IF Tx=1 AND J=250 THEN GOSUB Temp

wData=$700

GOSUB Send

I2CREAD Dpin,Cpin,$A3,$04,[T]

T=T&$3F

GOSUB Tcon

GOSUB vcon

for C=0 to 2                   ' Read data out to STV5730A

LOOKUP2 C,[CS[0],CS[1],$26],Co

wData=$1300+Co

gosub Send

NEXT C

I2CREAD Dpin,Cpin,$A3,$03,[T]

GOSUB Tcon

GOSUB vcon

FOR C=0 to 2                   ' Read data out to STV5730A

LOOKUP2 C,[CS[0],CS[1],$26],Co

wData=$1300+Co

GOSUB Send

NEXT C

I2CREAD Dpin,Cpin,$A3,$02,[T]

GOSUB Tcon

GOSUB vcon

FOR C=0 to 1                   ' Read data out to STV5730A

LOOKUP2 C,[CS[0],CS[1]],Co

wData=$1300+Co

gosub Send

NEXT C

Hserin 40,v_time,[echo]

IF ECHO=27  THEN

GOSUB linefeed

Hserout ["Exit"]

GOSUB linefeed

GOSUB linefeed

GOSUB CLS

CC=0: Lc=0 : L=0 : Cx=0 : Tx=0

RETURN

ENDIF

GOTO v_time:

v_date:                        ' Display date via the video

wData=$800

gosub Send

I2CREAD Dpin,Cpin,$A3,$05,[T]

T=T&$3F

SumA=t

GOSUB Tcon

GOSUB vcon

for C=0 to 2                   ' Read data out to STV5730A

LOOKUP2 C,[CS[0],CS[1],$28],Co

wData=$1300+Co

GOSUB Send

next C

I2CREAD Dpin,Cpin,$A3,$07,[T]

T=T&$1F

GOSUB Tcon

GOSUB vcon

for C=0 to 2                   ' Read data out to STV5730A

LOOKUP2 C,[CS[0],CS[1],$28],Co

wData=$1300+Co

GOSUB Send

next C

I2CREAD Dpin,Cpin,$A3,$08,[T]

GOSUB Tcon

GOSUB vcon

FOR C=0 to 3                   ' Read data out to STV5730A

LOOKUP2 C,[2,0,CS[0],CS[1]],Co

wData=$1300+Co

GOSUB Send

NEXT C

RETURN

Tempcom:                       ' Read temperture

GOSUB Tmp1

IF D[3]=$0B THEN

D[3]=32

ELSE

D[3]=D[3]+H

ENDIF

IF D[2]<$0A THEN D[2]=D[2]+H

IF D[2]=$0B THEN D[2]=32

IF D[2]=$0A THEN D[2]=45

Hserout ["temp 1 ",D[2],D[3],D[4]+H,"C"]

GOSUB linefeed

GOSUB Tmp2

IF D[3]=$0B THEN

D[3]=32

ELSE

D[3]=D[3]+H

ENDIF

IF D[2]<$0A THEN D[2]=D[2]+H

IF D[2]=$0B THEN D[2]=32

IF D[2]=$0A THEN D[2]=45

Hserout ["temp 2 ",D[2],D[3],D[4]+H,"C"]

GOSUB linefeed

GOSUB linefeed

RETURN

Temp:                      ' Display Temperture via the video

wData=$70C

gosub Send

for C=0 to 10              ' Read data out to STV5730A

LOOKUP2 C,[$1F,$2E,$36,$39,$2E,$3B,$2A,$3D,$3E,$3B,$2E],Co

wData=$1300+Co

gosub Send

next C

GOSUB Tmp1

wData=$80C

gosub Send

for C=0 to 5             ' Read data out to STV5730A

LOOKUP2 C,[$1,$0B,D[2],D[3],D[4],$0E],Co

wData=$1300+Co

gosub Send

next C

GOSUB Tmp2

wData=$814

gosub Send

for C=0 to 5             ' Read data out to STV5730A

LOOKUP2 C,[$2,$0B,D[2],D[3],D[4],$0E],Co

wData=$1300+Co

gosub Send

next C

Tx=1

J=0

RETURN

Tmp1:

ADCIN 0,AD0        ' Read ADC one

Tp=AD0/64

GOSUB Temp_C

RETURN

Tmp2:

ADCIN 1,AD1        ' Read ADC two

Tp=AD1/64

GOSUB Temp_C

RETURN

Temp_C:

IF Tp>=559 THEN

J=(Tp-559)*100     ' Calulate out temperture in +

C=J/205

ELSE

J=(Tp-354)*100     ' Calulate out temperture in -

C=100-(J/205)

ENDIF

GOSUB AtoDcaL

RETURN

AtoDcaL:

D[4]=C DIG 0     ' Read the position of digit 5

D[3]=C DIG 1     ' Read the position of digit 4

D[2]=C DIG 2     ' Read the position of digit 3

D[1]=C DIG 3     ' Read the position of digit 2

D[0]=C DIG 4     ' Read the position of digit 1

IF D[0]+D[1]+D[2]+D[3]=0 THEN D[3]=$0B   ' Add space in (Hex B)

IF D[0]+D[1]+D[2]=0 THEN D[2]=$0B

IF D[0]+D[1]=0 THEN D[1]=$0B

IF D[0]=0 THEN D[0]=$0B

RETURN

' Comport setup

com_set:

wData=0 : Sz=0

L=0 : TAB=0 : J=0

CC=0: Lc=0 : Cx=0 : Tx=0

hserout ["Video overlay"]    ' Send text followed by carriage return and linefeed

GOSUB linefeed

Hserout ["by Grant VE3XTV"]	 ' Send text followed by carriage return and linefeed

GOSUB linefeed               ' Carriage return and linefeed

com_start:     ' Comport start up

IF Sz=0 THEN

wData=$00CC

GoSub Send

wData=$1000

GoSub Send

ENDIF

IF Sz=1 THEN

wData=$00CC

GoSub Send

wData=$1555

GoSub Send

ENDIF

IF Sz=2 THEN

wData=$00CC

GoSub Send

wData=$1AAA

GoSub Send

ENDIF

IF Sz=3 THEN

wData=$00CC

GoSub Send

wData=$1FFF

GoSub Send

ENDIF

com:

FOR NC=1 TO 10

Hserin 4000,com_start,[echo]	     ' Get a echo from serial port

IF ECHO=13 THEN

GOTO getinfo

GOTO com

ENDIF

IF NC=1  THEN  D[0]=ECHO               ' Store last 10 ASCII digits

IF NC=2  THEN  D[1]=ECHO

IF NC=3  THEN  D[2]=ECHO

IF NC=4  THEN  D[3]=ECHO

IF NC=5  THEN  D[4]=ECHO

IF NC=6  THEN  D[5]=ECHO

IF NC=7  THEN  D[6]=ECHO

IF NC=8  THEN  D[7]=ECHO

IF NC=9  THEN  D[8]=ECHO

IF NC=10 THEN D[9]=ECHO

Hserout [echo]		                 ' Send echo out serial port

NEXT NC

Goto com

getinfo:

GOSUB linefeed

IF D[0]=67 AND D[1]=76 AND D[2]=83 THEN                   ' CLS Clear screen

GOSUB CLS

GOSUB linefeed

Hserout ["Clear screen"]

GOSUB linefeed

GOSUB linefeed

CC=0: Lc=0 : L=0 : Cx=0 : Tx=0

GOTO com

ENDIF

IF D[0]=67 THEN                                      ' C?????? set callsgin

CS[0]=D[1] : CS[1]=D[2] : CS[2]=D[3] : CS[3]=D[4] : CS[4]=D[5] : CS[5]=D[6]

GOSUB linefeed

Hserout ["Set to ",CS[0],CS[1],CS[2],CS[3],CS[4],CS[5]]

GOSUB linefeed

GOSUB linefeed

GOTO com

ENDIF

IF D[0]=68 AND D[1]=67 THEN                            ' DC display callsgin

GOSUB DC

GOSUB linefeed

Hserout ["Display callsgin"]

GOSUB linefeed

GOSUB linefeed

GOTO com

ENDIF

IF D[0]=84 AND D[1]=89 AND D[2]=80 AND D[3]=69 THEN        ' TYPE keyboard mode

GOSUB linefeed

Hserout ["Keyboard mode"]

GOSUB linefeed

GOSUB linefeed

GOTO TYPE

ENDIF                                              ' Font size=1

IF D[0]=72 AND D[1]=49 THEN

Sz=0

GOSUB linefeed

Hserout ["Font size=1"]

GOSUB linefeed

GOSUB linefeed

ENDIF

IF D[0]=72 AND D[1]=50 THEN

Sz=1

GOSUB linefeed

Hserout ["Font size=2"]                      ' Font size=2

GOSUB linefeed

GOSUB linefeed

ENDIF

IF D[0]=72 AND D[1]=51 THEN

Sz=2

GOSUB linefeed

Hserout ["Font size=3"]                      ' Font size=3

GOSUB linefeed

GOSUB linefeed

ENDIF

IF D[0]=72 AND D[1]=52 THEN

Sz=3

GOSUB linefeed

Hserout ["Font size=4"]                      ' Font size=4

GOSUB linefeed

GOSUB linefeed

ENDIF

IF D[0]=82 AND D[1]=68 THEN                      ' 82 and 68 is RD to read callsgin

GOSUB linefeed

GOSUB Rdata

ENDIF

IF D[0]=87 AND D[1]=68 THEN                      ' 82 and 68 WD write data

GOSUB linefeed

GOSUB Memstore

Hserout ["Writing data"]

GOSUB linefeed

GOSUB linefeed

GOTO com

ENDIF

IF D[0]=84 AND D[1]=73 AND D[2]=77 AND D[3]=69 THEN  ' TIME display time on PC

GOSUB linefeed

Hserout ["Time is"]

GOSUB linefeed

GOSUB linefeed

GOSUB PC_time

GOTO com

ENDIF

IF D[0]=68 AND D[1]=65 AND D[2]=84 AND D[3]=69 THEN  ' DATE display time on PC

GOSUB linefeed

Hserout ["Date is"]

GOSUB linefeed

GOSUB linefeed

GOSUB PC_date

GOTO com

ENDIF

IF D[0]=83 AND D[1]=84 THEN                      ' ST Set time format "hh:mm" enter to set

T10=D[2] : T1=D[3]

GOSUB Set_time

I2CWRITE Dpin,Cpin,$A2,$04,[Co]

T10=D[5] : T1=D[6]

GOSUB Set_time

I2CWRITE Dpin,Cpin,$A2,$03,[Co]

GOSUB linefeed

Hserout [D[2],D[3],":",D[5],D[6]]

GOSUB linefeed

GOSUB linefeed

GOTO com

ENDIF

IF D[0]=83 AND D[1]=83 THEN                      ' SS Set seconds format "ss" enter to set

GOSUB linefeed

Hserout [D[2],D[3]," seconds"]

GOSUB linefeed

GOSUB linefeed

T10=D[2] : T1=D[3]

GOSUB Set_time

I2CWRITE Dpin,Cpin,$A2,$02,[Co]

GOTO com

ENDIF

IF D[0]=83 AND D[1]=68 THEN                      ' SD Set date format "dd/MM/YY" enter to set

T10=D[2] : T1=D[3]

GOSUB Set_time

I2CWRITE Dpin,Cpin,$A2,$05,[Co]

T10=D[5] : T1=D[6]

GOSUB Set_time

I2CWRITE Dpin,Cpin,$A2,$07,[Co]

T10=D[8] : T1=D[9]

GOSUB Set_time

I2CWRITE Dpin,Cpin,$A2,$08,[Co]

GOSUB linefeed

Hserout [D[2],D[3],"/",D[5],D[6],"/",D[8],D[9]]

GOSUB linefeed

GOSUB linefeed

GOTO com

ENDIF

IF D[0]=83 AND D[1]=87 THEN                     ' SW Set week day format "w" enter to set

GOSUB linefeed

Hserout [D[2]," day of the week"]

GOSUB linefeed

GOSUB linefeed

T10=0 : T1=D[2]

GOSUB Set_time

I2CWRITE Dpin,Cpin,$A2,$06,[Co]

GOTO com

ENDIF

IF D[0]=68 AND D[1]=84 AND D[2]<>77 AND D[3]<>80 THEN  ' DT Display time via video

J=0

GOSUB linefeed

Hserout ["Display time"]

GOSUB linefeed

GOSUB linefeed

GOSUB v_time

ENDIF

IF D[0]=68 AND D[1]=68 THEN                        ' DD Display date via video

Cx=1

GOSUB linefeed

Hserout ["Display date"]

GOSUB linefeed

GOSUB linefeed

GOSUB v_date

ENDIF

IF D[0]=68 AND D[1]=84 AND D[2]=77 AND D[3]=80 THEN    ' DTMP display temperature via video

Tx=1

GOSUB linefeed

Hserout ["Display temperature"]

GOSUB linefeed

GOSUB linefeed

GOSUB temp

ENDIF

IF D[0]=84 AND D[1]=69 AND D[2]=77 AND D[3]=80 THEN    ' TEMP display temperature

GOSUB linefeed

Hserout ["Temperature is"]

GOSUB linefeed

GOSUB linefeed

GOSUB Tempcom

GOTO com

ENDIF

IF D[0]=72 AND D[1]=69 AND D[2]=76 AND D[3]=80 THEN    ' HELP infomation

GOSUB linefeed

Hserout ["CLS clear display"]

GOSUB linefeed

Hserout ["DC display callsgin"]

GOSUB linefeed

Hserout ["DTMP display temperature"]

GOSUB linefeed

Hserout ["DT display time"]

GOSUB linefeed

Hserout ["DD display date"]

GOSUB linefeed

Hserout ["H1,H2,H3,H4 font sizes 1x up to 4x"]

GOSUB linefeed

Hserout ["TYPE keyboard text mode, video overlay"]

GOSUB linefeed

Hserout ["TIME display time on PC"]

GOSUB linefeed

Hserout ["DATE display date on PC"]

GOSUB linefeed

Hserout ["TEMP display temperature on PC"]

GOSUB linefeed

GOSUB linefeed

Hserout ["****** Setup information ******"]

GOSUB linefeed

Hserout ["RD read data"]

GOSUB linefeed

Hserout ["WD write data"]

GOSUB linefeed

Hserout ["C?????? set callsgin"]

GOSUB linefeed

Hserout ["SS set seconds format 'ss'"]

GOSUB linefeed

Hserout ["ST set time format 'hh:mm'"]

GOSUB linefeed

Hserout ["SD set date format 'dd/MM/YY'"]

GOSUB linefeed

Hserout ["SW set week day format 'w'"]

GOSUB linefeed

GOSUB linefeed

GOTO com

ENDIF

GOTO com_start

Rdata:

GOSUB linefeed

Hserout ["Font size=",DEC Sz+1]

GOSUB linefeed

GOSUB Memread

GOSUB linefeed

Hserout ["Callsgin=",CS[0],CS[1],CS[2],CS[3],CS[4],CS[5]]    ' Send message

GOSUB linefeed

GOSUB linefeed

GOSUB linefeed

rETURN

Memstore:             ' Store Callsign

FOR C=0 TO 5

WRITE C,CS[C]

NEXT C

RETURN

Memread:              ' Read Callsign

FOR C=0 TO 5

READ C,CS[C]

NEXT C

RETURN

DCLS:                 ' Clear the screen set to ' '

FOR C=0 TO 27

D[C]=32

NEXT C

RETURN

com1:

GOSUB DCLS

FOR NC=1 TO 28

Hserin 4000,com1,[echo]	            ' Get a echo from serial port

IF ECHO=13 THEN GOTO TYPE1

IF ECHO=9 THEN GOSUB Tabspace

IF ECHO=92 THEN

GOSUB CLS : CC=0: Lc=0 : L=0

GOTO com1 : ENDIF

IF ECHO=8 THEN

GOSUB Backspace : ENDIF

IF ECHO=27  THEN

GOSUB linefeed

Hserout ["Exit"]

GOSUB linefeed

GOSUB linefeed

GOTO com : ENDIF

IF NC=1  THEN  D[0]=ECHO               ' Store last 28 ASCII digits

IF NC=2  THEN  D[1]=ECHO

IF NC=3  THEN  D[2]=ECHO

IF NC=4  THEN  D[3]=ECHO

IF NC=5  THEN  D[4]=ECHO

IF NC=6  THEN  D[5]=ECHO

IF NC=7  THEN  D[6]=ECHO

IF NC=8  THEN  D[7]=ECHO

IF NC=9  THEN  D[8]=ECHO

IF NC=10 THEN D[9]=ECHO

IF NC=11 THEN D[10]=ECHO

IF NC=12 THEN D[11]=ECHO

IF NC=13 THEN D[12]=ECHO

IF NC=14 THEN D[13]=ECHO

IF NC=15 THEN D[14]=ECHO

IF NC=16 THEN D[15]=ECHO

IF NC=17 THEN D[16]=ECHO

IF NC=18 THEN D[17]=ECHO

IF NC=19 THEN D[18]=ECHO

IF NC=20 THEN D[19]=ECHO

IF NC=21 THEN D[20]=ECHO

IF NC=22 THEN D[21]=ECHO

IF NC=23 THEN D[22]=ECHO

IF NC=24 THEN D[23]=ECHO

IF NC=25 THEN D[24]=ECHO

IF NC=26 THEN D[25]=ECHO

IF NC=27 THEN D[26]=ECHO

IF NC=28 THEN D[27]=ECHO

Hserout [echo]		                 ' Send echo out serial port

NEXT NC

Goto com1

Tabspace:

TAB=TAB+7

FOR C=1 TO 7

Echo=32

Hserout [echo]

NEXT C

RETURN

Backspace:

NC=NC-1

IF NC<=1 THEN NC=1

LOOKUP2 Nc,[0,D[0],D[1],D[2],D[3],D[4],D[5],D[6],D[7],D[8],D[9],D[10],D[11],D[12],D[13],D[14],D[15],D[16],D[17],D[18],D[19],D[20],D[21],D[22],D[23],D[24],D[25],D[26],D[27]],Co

Co=Echo

Hserout [echo]

Hserin 2000,com_start,[echo]	     ' Get a echo from serial port

IF ECHO=8 THEN GOTO Backspace

RETURN

DC:                ' Display Callsgin

GOSUB DCLS

GOSUB Memread

FOR C=0 TO 5

AS=CS[C]

GOSUB Convert

D[C]=AS

NEXT C

X=1

GOSUB Display

X=0

RETURN

TYPE:                      ' Conversion for the video overlay

GOSUB linefeed

GOTO com1

TYPE1:

GOSUB linefeed

AS=D[0]

GOSUB Convert

D[0]=AS

AS=D[1]

GOSUB Convert

D[1]=AS

AS=D[2]

GOSUB Convert

D[2]=AS

AS=D[3]

GOSUB Convert

D[3]=AS

AS=D[4]

GOSUB Convert

D[4]=AS

AS=D[5]

GOSUB Convert

D[5]=AS

AS=D[6]

GOSUB Convert

D[6]=AS

AS=D[7]

GOSUB Convert

D[7]=AS

AS=D[8]

GOSUB Convert

D[8]=AS

AS=D[9]

GOSUB Convert

D[9]=AS

AS=D[10]

GOSUB Convert

D[10]=AS

AS=D[11]

GOSUB Convert

D[11]=AS

AS=D[12]

GOSUB Convert

D[12]=AS

AS=D[13]

GOSUB Convert

D[13]=AS

AS=D[14]

GOSUB Convert

D[14]=AS

AS=D[15]

GOSUB Convert

D[15]=AS

AS=D[16]

GOSUB Convert

D[16]=AS

AS=D[17]

GOSUB Convert

D[17]=AS

AS=D[18]

GOSUB Convert

D[18]=AS

AS=D[19]

GOSUB Convert

D[19]=AS

AS=D[20]

GOSUB Convert

D[20]=AS

AS=D[21]

GOSUB Convert

D[21]=AS

AS=D[22]

GOSUB Convert

D[22]=AS

AS=D[23]

GOSUB Convert

D[23]=AS

AS=D[24]

GOSUB Convert

D[24]=AS

AS=D[25]

GOSUB Convert

D[25]=AS

AS=D[26]

GOSUB Convert

D[26]=AS

AS=D[27]

GOSUB Convert

D[27]=AS

GOSUB Display

GOTO com1

Convert:                           ' Convert from ACSII to TROM data of the STV5730A

IF AS>=48 AND AS<=57  THEN

AS=AS-48

RETURN

ENDIF

IF AS>=65 AND AS<=90  THEN

AS=AS-53

RETURN

ENDIF

IF AS>=97 AND AS<=122 THEN

AS=AS-55

RETURN

ENDIF

IF AS>=46 AND AS<=47  THEN

AS=AS-7

RETURN

ENDIF

IF AS=45 THEN

AS=$A

RETURN

ENDIF

IF AS=32 THEN

AS=$B

RETURN

ENDIF

IF AS=34 THEN

AS=$6F

RETURN

ENDIF

IF AS=58 THEN

AS=$26

RETURN

ENDIF

IF AS=39 THEN

AS=$29

RETURN

ENDIF

IF AS=42 THEN

AS=$5F

RETURN

ENDIF

IF AS=43 THEN

AS=$5E

RETURN

ENDIF

IF AS=63 THEN

AS=$70

RETURN

ENDIF

IF AS=60 THEN

AS=$7B

RETURN

ENDIF

IF AS=62 THEN

AS=$7A

RETURN

ENDIF

IF AS=94 THEN

AS=$7C

RETURN

ENDIF

IF AS=124 THEN

AS=$7D

RETURN

ENDIF

IF AS=35 THEN

AS=$6E

RETURN

ENDIF

IF AS=36 THEN

AS=$71

RETURN

ENDIF

IF AS=37 THEN

AS=$79

RETURN

ENDIF

IF AS=64 THEN

AS=$45

RETURN

ENDIF

IF AS=61 THEN

AS=$78

ELSE

AS=$B

ENDIF

RETURN

Display:                              ' Font size 1 to 4

IF Sz=0 THEN GOSUB Size_1

IF Sz=1 THEN GOSUB Size_2

IF Sz=2 THEN GOSUB Size_3

IF Sz=3 THEN GOSUB Size_4

RETURN

Size_1:

wData=L+tab

gosub Send

IF X=0 THEN

J=27

ELSE

J=5

ENDIF

FOR C=0 TO J

LOOKUP2 c,[D[0],D[1],D[2],D[3],D[4],D[5],D[6],D[7],D[8],D[9],D[10],D[11],D[12],D[13],D[14],D[15],D[16],D[17],D[18],D[19],D[20],D[21],D[22],D[23],D[24],D[25],D[26],D[27]],Co

wData=$1300+Co

gosub Send

NEXT C

L=L+$100

IF L=$B00 THEN L=0

TAB=0

RETURN

Size_2:

wData=L+tab

gosub Send

FOR C=0 TO 13

LOOKUP2 c,[D[0],D[1],D[2],D[3],D[4],D[5],D[6],D[7],D[8],D[9],D[10],D[11],D[12],D[13]],Co

wData=$1300+Co

gosub Send

NEXT C

L=L+$100

IF L=$600 THEN L=0

TAB=0

RETURN

Size_3:

wData=L+tab

gosub Send

FOR C=0 TO 9

LOOKUP2 c,[D[0],D[1],D[2],D[3],D[4],D[5],D[6],D[7],D[8],D[9]],Co

wData=$1300+Co

gosub Send

NEXT C

L=L+$100

IF L=$400 THEN L=0

TAB=0

RETURN

Size_4:

wData=L+tab

gosub Send

FOR C=0 TO 6

LOOKUP2 c,[D[0],D[1],D[2],D[3],D[4],D[5],D[6]],Co

wData=$1300+Co

gosub Send

NEXT C

L=L+$100

IF L=$300 THEN L=0

TAB=0

RETURN

linefeed:

Hserout [13,10]

return